Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	MASS TRANSFER OPERATIONS					              Time : 3 hours

Subject Code: 	BT236			         						                Maximum Marks: 100

#### (Allowed the mass transfer data book and extra graph sheets)

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	State the Raoults law and Henrys law.

2.	Write the Stefan’s equation.

3.	Raleigh equation is used for _________.

4.	What are the methods available to determine the number of stages?

5.	What are the types of adsorption

6.	List out the various Adsorbents

7.	What are the types of absorption?

8.	The absorption factor is defined as_________.

9.	_________ is used for extraction of oil seed.

10.	Coffee is leached from coffee beans by leaching with _________.

PART – B  (5 x 3 = 15 MARKS)

11.	Define: Mass flux and Fick’s law

12.	Define: Murphee plate efficiency, Point efficiency and Over all efficiency

13.	Write the characters of solvents.

14.	Differentiate between the physical and chemical adsorptions.

15.	Define extraction and leaching

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Discuss about the molecular diffusion of “A” through Non –diffusing “B”    under steady

state conditions.    				    											(7)

b.	A tube of small diameter was filled with acetone (density = 0.79  g-cm-3) up to  1.10  cm  from top and maintained at a temperature  20 °C in a gentle current air after 5 hrs the level of liquid fell to 2.05 cm from the top. Calculate the diffusivity of acetone in air, if the barometric pressure was 750 mmHg Vapor pressure of Acetone at 20 °C is

180 mm Hg.  																(8)

(OR)

17.	a.	Explain inter-phase Mass Transfer – two film theory.                					(5)

b.	Oxygen (A) is diffusing through Carbon monoxide (B) under steady state conditions with

Carbon monoxide non diffusing .The total pressure is 1 atm pressure and temperature is

0 °C. The partial pressure of oxygen at two planes 0.2 cm apart is respectively 100 and

50 mm Hg .The diffusivity for the mixture is 0.185 cm3/ sec. 						(10)

18.	a. 	Derive the Raleigh   equation with neat sketch.                              					(5)

b. 	Discuss the Azeotropic distillation and Extractive distillations with neat diagram.   	(10)

(OR)

19. 	a.	A  mixture of benzene – Toluene  containing  40 mole % benzene is to be separated  to give

a product  of 90 mole % benzene at the top and the bottom product contain  not more than 10 mole % benzene. Using an average value of 2.4 for volatility of benzene relative to toluene. Calculate: (a) The number of theoretical plates required (b) Find the location of feed plate. If the feed is liquid at its boiling point. The reflux is 2.5 times of minimum reflux ratio.    																		(8)

b.	A liquid feed at its boiling point at 60 °C is fed into the distillation column at the rate of

400 kgmole / hr containing 60 mole % Benzene and 40 mole % toluene at a pressure of 101.3 Kpa. The bottom product flow is to be 60 kg mole / hr containing 10 % benzene and no toluene. Calculate the composition of distillate and the number of theoretical plates at total reflux.                                												(7)

| X | 1.0 | 0.78 | 0.581 | 0.211 | 0.258 | 0.130 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Y | 1.0 | 0.9 | 0.777 | 0.532 | 0.456 | 0.261 | 0 |

20. 	a.	Discuss about the design of Gas absorption tower.                      					(10)

b.	Discuss about the packing methods.                                                 					(5)

(OR)

| 21. Ammonia – air mixture containing 2 % Ammonia at 20 °C and 1 atmosphere is to be scrubbed with water in a sieve plate column. The water rate is 32.6 kg / square meter sec. 98 % Ammonia is to be recovered .Calculate the number of ideal stages assuming 1.5 times that of min flow rate, isothermal operation. (Constant temperature) PA mmHg | 12 | 18.1 | 31.7 | 50 | 69.6 | 10 |
| --- | --- | --- | --- | --- | --- | --- |
| CA gm of NH3/ 100gm of Water | 2 | 3 | 5 | 7.5 | 10 | 20 |

22. 	a. 	Explain the single stage and two stage adsorption operation with neat sketch.           	(10)

b.	Discuss about briefly the principles of Ion Exchanger.	            					(5)

(OR)

23. 	A solid adsorbent is used to remove colored impurities from an aqueous solution. The original value of color on an arbitrary scale .It is required to reduce this value to 10 % of its original value 48.Using the following data. Find the quantity of fresh adsorbent used for 1000 kg of solution for

a.	Single stage operation process

b.	Two stage operation process.  The intermediate value is 24.

24. 	Describe with a neat sketch the design and the working principle of a commercial extraction units

a.	Boll man extractor / basket extractor.

b     Rotocel extractor

(OR)

25. 	a.	Define Leaching and give examples.		                       						(5)

b.	Describe about the leaching of fine solids with neat sketch.        					(10)