Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	ADVANCED BIOPROCESS ENGINEERING              Time : 3 hours

Subject Code: 	BT303			         						               Maximum Marks: 100

#### Answer ALL questions (5 x 20 = 100 Marks)

1.	a.	Power requirement of mixing gassed system is always less than the ungassed system –

reason out with explanation. 												(10)

b.	Explain the film and surface renewal theory of diffusion mass transfer. 			(10)

(OR)

2.	a.	A liquid medium initially contains bacterial spores at a concentration of 9.2 X 1011 m-3 . The

medium is sterilized thermally at 125˚C and spore density is recorded. The data are as follows. (10)

| Time (min) | 0 | 5 | 10 | 20 | 30 | 35 |
| --- | --- | --- | --- | --- | --- | --- |
| Spore density (m-3) | 9.2X1011 | 2X1010 | 3.32X109 | 2.3X105 | 115 | 3 |

i.	Find the thermal death kinetic rate constant in s-1.

ii.	Find the inactivation factor after 40 min

b.	List out the advantages and disadvantages of various sterilization methods. 		(10)

3. 	a.	Explain the Lotka – Volterra Model for predator – prey oscillations. 				(10)

b.	Consider a scale up from 10 to 10000 liters. The vessel height to the diameter is 0.25 times the vessel diameter. The agitator rpm is 300 in small scale. Determine the dimensions of the large scale fermentor and agitator rpm for (i) Constant power per unit volume; (ii) constant NRe ; (iii) for constant pumping rate per volume and (iv) for constant maximum impeller shear. 	(10)

(OR)

4. 	a.	Explain with neat diagram, the basic design and operating principle of stainless steel

insitu sterilizable 1000 L fermentor with its ancillaries.  						(15)

b.	List out the scale up rules for agitator design. 								 (5)

5.	Compulsory:

a.	Calculate the steady state substrate and biomass concentrations in a continuous fermentor which has an operating volume of 25 L when the sterile feed stream contains limiting substrate at 2000 mg/L and enters the vessel at 8 L/h. The values of Ks and µm are 10.5 mg/L and 0.45 h-1 respectively and the yield coefficient may be taken to be 0.48. 								(10)

b.	Explain the enzyme deactivation kinetics 									(10)

6. 	Explain simple unstructured and structured kinetic models for the growth of filamentous organisms.

(OR)

7. 	a.	Explain the growth associated product formation kinetics. 						(10)

b.	Discuss the design and analysis of packed bed bioreactors. 					(10)

8. 	Explain the operating principle of the following online sensors:

a.	Galvanic & Polarographic types of Dissolved Oxygen probe.   b.	pH electrode.

(OR)

9.	Explain the applications of Artificial Intelligence and Neural Network in bioreactor control

with a case study.