Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	MICROPROCESSORS								              Time : 3 hours

Subject Code: 	CA242			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is a bus?

2.	What are the basic units of a microprocessor?

3.	What is an assembler?

4.	What is program counter?

5.	What is instruction cycle?

6.	Give an example instruction (8085) for implicit addressing mode.

7.	What is an interrupt?

8.	What is the function performed by DI instruction?

9.	TRAP is a maskable interrupt. State true or false.

10.	What is DMA data transfer?

PART – B  (5 x 3 = 15 MARKS)

11.	Why data bus is bidirectional?

12.	What is a flag?

13.	Explain RET instruction

14.	How the interrupt are classified?

15.	Write a short note on DMA.

PART – C  (5 x 15 = 75 MARKS)

16.	Draw and detail the pin diagram of 8085.

(OR)

17.	What are the features of a microcomputer that distinguish it from a general purpose computer? Explain.

18.	a.	Design a memory interfacing circuit for 8085 using the following.

1 K x 8 RAM and  2 numbers of 8 K x 8 ROM									(8)

b.	Design a suitable interfacing circuit to interface the seven LED display.    			(7)

(OR)

19.	a.	Compare memory Mapped I/O and I/O mapped I/O.								(5)

b.	Absolute address decoding and partial address decoding.							(10)

20.	Explain the various addressing modes of 8085 with examples

(OR)

21.	Using the 8085 programming language, write the assembly program to add the 16-bit number in memory locations 200016 and 200116 to the 16-bit number in memory locations 200216 and 200316. The most significant 8 bits of the two numbers to be added are in memory locations 200116 and 200316. Store result in DE.

22.	Describe organization of 8259 programmable interrupt controller

(OR)

23.	Write short notes on:															(5+5+5)

a.	Counter 8253		b.	Subroutines and				c.	 Stack

24.	Draw and explain the internal block diagram for 8255.

(OR)

25.	a.	With relevant block diagram explain the organization of 8251 USART.				(7)

b.	Describe the data transfer DMA cycle of 8085.									(8)