Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	FLUID MECHANICS AND MACHINERY			               Time : 3 hours

Subject Code: 	CE212			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	State Newton’s Law of viscosity.

2.	What is unit of kinematic viscosity in SI system?

3.	Stare Pascal’s Law.

4.	How will you distinguish Laminar flow from turbulent flow based on Reynold’s Number?

5.	Write down the expression for Loss of head due to sudden enlargement in pipe flow.

6.	How will you connect multi-stage centrifugal pump to get more discharge?

7.	What do you mean by ‘slip’ in case of a reciprocating pump?

8.	Define: ‘pitch ratio’ in a pelton wheel turbine.

9.	What is a Draft tube?

10.	One atmospheric pressure head is equal to _______mts. of water.

PART – B  (5 x 3 = 15 MARKS)

11.	Derive an expression for surface tension of liquid Droplet.

12.	Differentiate between rotational and irrotational flow.

13.	What are the two conditions to be satisfied in flow of fluid through pipes laid in parallel?

14.	What are the various methods of determining the manometric Head in case of a centrifugal pump?

15.	How will you classify the turbines?

PART – C  (5 x 15 = 75 MARKS)

16.	Two coaxial cylinders 100mm and 975mm in diameter and 2.5cm high have both their ends open and have a viscous fluid filled in between. A torque of 1.2 N – m is produced on the inner cylinder when the outer one rotates at 90 rpm. Determine the coefficient of viscosity of the liquid.

(OR)

17.	The Lower corner of a water tank has shape of quadrant of a circle of radius 1.2m. The water surface is 2.4m above the centre of curvature. The water tank is 3m long. Find the magnitude, direction and location of the total force extended by the water surface on this curved surface.

18.	a.	For the following flow, determine the components of rotation about the various axes.		(8)

U = xy3z 	:  = -y2 z2	:	w = yz2−y3z2z

b.	Obtain relevant stream function to the following set of velocity components of steady,

imcompressible flow. 	u= x + y	:	 = x – y.										(7)

(OR)

19.	Derive an expression for Euler’s equation of motion and hence deduce the Bernoulli’s equation, stating the assumptions made

[P.T.O]

20.	In a 45bend a rectangular air duct of 1m3 cross-sectional area is gradually reduced to 0.5m2 area. Find the magnitude and direction of force required to hold the duct in position if the velocity of flow at 1m2 section is 10 m/s and the pressure is 30 kN/m2. Take the specific weight of air as 0.0116 kN/m3.

(OR)

21.	Two sharp ended pipes of diameters 5cm and 10cm respectively each of length 100m respectively are connected in parallel between two reservoirs which have a difference of level of 10m. If the friction factor for each pipe is 0.32, calculate

a.	Rate of flow of each pipe.

b.	The diameter of a single pipe 100m long which would give the same discharge, if it were

substituted for the original two pipes.

22.	A centrifugal pump running at 800 r.p.m is working against a total head of 20.2m. The external diameter of the impeller is 480mm and outlet width 60mm. If the vane angle at outlet is 40 and

manometric efficiency is 70%, determine.

- velocity of flow at outlet.

- Absolute velocity of water learning the vane.

- Angle made by absolute velocity at outlet with the direction of motion at outlet.

- Rate of flow through the pump.

(OR)

23.	With a neat sketch explain the working principle of a single acting reciprocating pump.

24.	A single jet pelton wheel runs at 300 r.p.m under ahead of 510m. The jet diameter is 200mm, its deflection inside the bucket is 165 and its relative velocity is reduced by 15% due to friction. Determine.

a.	water power

b.	resultant force on the bucket

c.	overall efficiency

Take C = 0.98	;	speed ratio = 0.46.

(OR)

25.	A in ward flow reaction turbine is supplied 0.233 m3/s of water under a head of 11m. The wheel vanes are radial at inlet and the inlet diameter is twice the outlet diameter. The velocity of flow is constant and equal to 1.83 m/s. The wheel makes 370 r.p.m, Determine:

a.	Guide vane angle.

b.	Inlet and outlet diameters of the wheel.

c.	The width of the wheel at inlet and exit.

Assume that the discharge is radial and there are no losses in wheel.