Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	BASIC STRUCTURAL DESIGN						              Time : 3 hours

Subject Code: 	CE235			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	When a load is transferred by the application of load through one surface to another surface in contact, the stress is known as _______________

2.	The ratio of yield stress to working stress of a material is denoted as _______________

3.	What is slenderness ratio?

4.	What is the type of load, a shear wall will take?

5.	What is safe bearing capacity?

6.	The flexural member designed to carry the load from brick masonry above an opening is termed as _______________

7.	Lack of wood on the edge or corner of a piece of timber is called _______________

8.	Preservation is to protect timber against _______________

9.	The thickness of a weld is specified as _______________

10.	For temporary structures, the type of structural connection used is _______________

PART – B  (5 x 3 = 15 MARKS)

11.	List out the various requirements to be considered in structural design.

12.	What do you mean by equivalent eccentricity of a masonry wall?

13.	What is a bed block?  Where it is used?

14.	How are defects in timber classified?

15.	What are the advantages of bolted connections?

PART – C  (5 x 15 = 75 MARKS)

16.	Discuss briefly the various methods of design of structures.  Illustrate the advantages and disadvantages of each method.

(OR)

17.	Explain the various types of loads to be accounted in structural design.

18.	Design an interior cross wall for a two storeyed building to carry 100 mm thick R.C.C. slabs with 3m ceiling height.  The wall is unstiffened and supports a 2.65m wide slab.  Live load on roof is 1.5 kN/m2.  Live load on floor is 2 kN/m2; Weight of 80 mm thick terrace is 1.96 kN/m2; Weight of floor finish is 0.8 kN/m2.

(OR)

19.	Design an interior wall of a two stroreyed building carrying concrete slab with storey height of 3m.  The wall is stiffened by 100 mm thick intersecting wall at 3600 mm c/c.  Also, the wall has a door opening of size 900 x 2000 at a distance of 200 mm from one of the intersecting walls.  Assume roof loading as 15 kN/m and floor loading as 12.5 kN/m.

[P.T.O]

20.	a.	What are the factors to be considered in fixing the depth of foundation?				(5)

b.	A wall carries a load of 100 kN/m including its own weight.  Find suitable width and depth

of foundation.  Assuming p = 90 kN/m2; w = 16 kN/m3 and φ = 300. 				(10)

(OR)

21.	Determine the area and minimum depth of the foundation of a column carrying a load of 700 kN including its own weight.  The gross safe bearing capacity of soil is 110 kN/m2.  The angle of response is 300 and the unit weight of soil is 16 kN/m2.

22.	A sal wood column is to be designed for an outside location with a selected grade timber.  If the axial load of column is 1000 kN and height of column is 3m.  Determine the size of section to be provided.

(OR)

23	Determine the safe axial load on a deodar wood column of size 300 x 300, excised location having unsupported length of (i) 2.80m (ii) 4.20m (iii) 6.20 m.

24.	A tie in a truss consists of 90 x 60 x 10 mm angles, welded on each side of a gusset plate 12mm thick through longer legs.  Design the welded joints.  Cross sectional area of each angle is        140 mm2.  Assume axial tensile stress as 150 N/mm2.

(OR)

25.	Design a suitable lap joint to connect two platter 100 x 8 and 100 x 12 mm.