Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	MECHANICS OF DEFORMABLE BODIES – I		              Time : 3 hours

Subject Code: 	CE240			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	State Hook’s law.

2.	A solid cylinder of certain material remains unchanged in volume when axially strained.

Determine the Poisson ratio of the material.

3.	The principal stresses at a point in a material are 30 N/mm2 in tension and 40 N/mm2 in compression. Find out the maximum shear stress.

4.	In a thin cylindrical vessel, the hoop stress developed in the walls is 50 N/mm2. Determine the longitudinal stress.

5.	Find the bending moment at mid span of BC of the overhanging beam shown in the figure. The beam is subjected to a uniformly distributed load of 10 kN/m.

2 m

4 m

2 m

A

B

C

D

6.	Determine the reaction at the fixed end of the cantilever shown in figure.

M

3m

7.	Draw the qualitative shear stress distribution diagram of a typical T section used as beam.

8.	What are flitched beams?

9.	Determine the torsional rigidity of a circular cross section of diameter 150 mm with modulus of rigidity 0.80 X 105 N/mm2.

10.	Find the power transmitted by a circular shaft subjected to a torque of 50000 N-m and 100 r.p.m in Watts.

PART – B  (5 x 3 = 15 MARKS)

11.	Find the load P and axial deformation of the member of diameter 40 mm shown in figure. Modulus of elasticity is 2 X 10 5 N/mm2.

500 mm

1000 mm

100 kN

50 kN

P

12.	The state of stress at a point is pure shear and the shear stress is 25 N/mm2. Determine the principal stresses.

13.	Draw the bending moment diagram of the simply supported beam shown in figure.

50 kN-m

3m

7m

14.	Determine the strain energy stored due to bending in the beam shown in figure.

L

W

EI

L

L/4

15.	A helical spring has a spring constant of 20000 kN/m. If a load of 1000 kN is applied along the axis of the spring, determine the axial deformation undergone by the spring.

PART – C  (5 x 15 = 75 MARKS)

16.	A rectangular block is subjected to forces as shown in figure. Modulus of elasticity of the material is 2 X 105 N/m2 and Poisson ratio 0.25. Determine (a) Change in volume (b) Bulk modulus and (c) Modulus of rigidity.

50 kN

100 mm

100 kN

30 kN

100 mm

500 mm

(OR)

17.	A steel bar of 50 mm diameter and 500 mm long is enclosed inside a copper tube of same length having external diameter 70 mm and internal diameter 60 mm. The composite section is heated through 50 0C. Determine the stresses in steel bar and copper tube. Coefficient of thermal expansion for steel 16 X 10 -6/ 0C and coefficient of thermal expansion for copper 12 X 10 -6/ 0C. Young’s modulus of steel 2 X 10 5 N/mm2 and Young’s modulus of copper 1 X 10 5 N/mm2

18.	The state of stress at a point in a material is shown in the diagram. Determine the  (a) Principal stresses (b) Orientation of principal planes (c) Maximum shear stress (d) Plane of maximum shear stress.

L/4

50 N/mm2

40 N/mm2

25 N/mm2

(OR)

19.	A thin cylindrical shell of diameter 2 m and length 2m is subjected to an internal pressure of 10 MPa. The thickness of the shell is 20 mm. Find the (a) circumferential stresses (b) longitudinal stresses (c) Change in volume. Take modulus of elasticity 2 X 105 N/mm2  and Poisson ratio as 0.25.

20.	Find the maximum negative and positive moments occurring in the overhanging beam shown in the figure along with their location from the left end A of the beam.

3m

7m

2m

A

B

C

D

18 kN/m

30 kN/m

2m

50 kN

100 kN

(OR)

21.	Find out the maximum bending moment and its location from the left end  of the beam shown in the figure.

50 kN/m

8 m

22. 	A simply supported beam of span 8 m is subjected to a uniformly distributed load of 60 kN/m

throughout its span. The ratio of the breadth to depth of the beam is 0.5. The maximum permissible compressive and tensile stresses are 10 N/mm2 and 4 N/mm2 respectively. Determine the beam dimensions.

(OR)

300 mm

450 mm

20 mm

30 mm

30 mm

23.	Determine the shear stresses at (a) junction of flange and web and (b) maximum shear stress of the I section shown in figure if it is subjected to shear force of 100 kN.

24.	A hollow shaft of diameter ratio 3/5 is required to transmit 600 kW power at 120 r.p.m. The maximum torque being 25% greater than that of mean torque. The shear stress is limited to        50 N/mm2 and the twist 0.0090 radjians per 1m.. Calculate the external diameter of the shaft. The modulus of rigidity of the shaft material is 0.80 X 10 5 N/mm2.

(OR)

25.	A stepped shaft consists of a solid shaft 70 mm diameter and length 1.0 m rigidly attached to a hollow shaft of internal diameter 70 mm and external diameter 120 mm and length 1.5m. Determine the maximum torque that can be applied without exceeding a shearing stress of        50 N/mm2 and a twist of 3 0 over a length of 2.5m. The modulus of rigidity is 0.8 X 10 5 N/mm2.