Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	MECHANICS OF DEFORMABLE BODIES - II		               Time : 3 hours

Subject Code: 	CE241			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The width of the Conjugate Beam is __________.

2.	Find the deflection of simply supported beam of span ‘l’ carries a point load W at its center.

3.	What are the types of failure of columns?

4.	Define slenderness ratio.

5.	In thick cylinder the circumferential stress in the wall thickness is _________________.

6.	Maximum Principal stress theory is also called __________________.

7.	Define shear centre.

8.	What are the two assumptions made in Winkler Bach theory?

9.	What are the methods used to locate Neutral axis?

10.	Under Unsymmetrical bending the resultant deflection of a beam is _____________.

PART – B  (5 x 3 = 15 MARKS)

11.	Define Fixed Beam and write any three advantages of fixed beam.

12.	Write the effective length for column with different end conditions.

13.	State maximum shear stress theory.

14.	A channel section has flanges 120 X 20 mm and web 160 X 10 mm. Determine the shear centre of the channel.

15.	Define Unsymmetrical bending? List any two reasons for unsymmetrical bending.

PART – C  (5 x 15 = 75 MARKS)

16.	A beam of 4 m span is simply supported at the end and is loaded as shown in fig.

below. Determine.	(i) 	Deflection at C

(ii) 	Maximum deflection, and

(iii) Slope at the end A

Take E=200X106kN/m2 and I=20X10−6 m4.

(OR)

17.	Using Conjugate beam method for the beam shown in fig. find the slope and deflection at A,B,C and D.

Take E=200X106kN/m2 and I=300X10−4 m4.

18.	a.	Derive the expression for crippling load when one end of the column is fixed and the other

end is hinged.																(12)

b.	Write the assumptions made in Euler’s theory.									(3)

(OR)

19.	A solid round bar 4 m long and 60 mm in diameter is used as a strut. Determine the Euler’s crippling load under the following end conditions.

a.	Both end Hinged				b.	One end fixed and the other end hinged

c.	Both ends are fixed, and 		d.	One end fixed and the other end free.

20.	Find the ratio of thickness of internal diameter for a tube subjected to internal pressure when the pressure is  5/8  of the value of the  maximum  permissible circumferential stress. Find the increase in internal diameter of such a tube 100mm internal diameter when the internal pressure is 80MN/mm2. Also find the changes in wall thickness. Take E=205x109N/m2 and 1/m=0.29.

(OR)

21.	a.	A  bolt is  under  an  axial  thrust of 9.6 kN together with a transverse force  of  4.8 kN.

Calculate the diameter according to:

i.	Maximum Principal stress theory		ii.	Maximum shear stress theory.

22.	Determine the position of the shear centre of the section of a beam shown in fig. below.

(OR)

23.	A crane hook carries a load of 5 kN the line of load being at a horizontal distance of 32 mm from the inside edge of a horizontal section through the centre of curvature; and the centre of curvature being 38 mm from the same edge. The horizontal section is a trapezium whose parallel sides are 13 mm and 26 mm and height is 32 mm. Determine the greatest tensile and compressive stresses in the hook.

24.	Calculate the stresses at points A,B and C of the cross section of the beam shown in fig. below.

(OR)

25.	A simply supported wooden beam 3 m long supports a total uniformly distributed load of 8 kN. The cross section is 100 mm X 150 mm. The applied load acts in a plane making an angle 30° with the vertical plane. Find the maximum bending stress developed in the beam. Self-weight of the beam may be neglected.