Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	ENGINEERING MECHANICS						              Time : 3 hours

Subject Code: 	CE242			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define Force.

2.	Define Free body diagram.

3.	If the resultant force on a rigid body is -------------, then the rigid body is in equilibrium.

4.	Two equal and opposite forces form a ---------.

5.	The polar moment of inertia of a circle of diameter   ‘d’ is------------------------.

6.	The moment of inertia of a triangle of base `b’ and height   ‘h’ about its base is ----------------.

7.	The time required to reach maximum height by a projectile having velocity of projection `u’ and angle of projection   ‘α’ is ------------------.

8.	------------of a rigid body is defined as the energy possessed by virtue of its position.

9.	The coefficient of restitution is defined as ----------------------------------------------.

10.	According to work energy principle, the work done by a system of forces acting on a body during translation is equal to ---------------------------.

PART – B  (5 x 3 = 15 MARKS)

11.	State Parallelogram law of forces.

12.	State the laws of dry friction.

13.	State Parallel and Perpendicular axis Theorem.

14.	What are the equations of rectilinear motion with constant acceleration?

15.	Distinguish between Direct Central impact and Oblique Central impact.

PART – C  (5 x 15 = 75 MARKS)

16. 	A particle  `O’ is subjected to the following concurrent  forces F 1  , F 2 , F 3 , F 4    and  F5   of     magnitude 100 N, 200 N, 300 N ,400 N and 500 N respectively .  All the forces act away from    the particle. The forces  F 1  , F 2 , F 3 , F 4    and  F5   make an angle of 30O  ,90O  ,120O  ,225O  and 0O       in the anticlockwise direction with the positive  x direction.  Determine the magnitude and  direction of the Resultant.

(OR)

17.	The resultant of two forces P and Q is 13 N if the angle between the two forces is 60O. If the angle between the two forces P and Q is 90O , the resultant force is 10 N. Determine (i) the forces P and Q (ii) the resultant  if the angle between the forces P and Q are 30O  and  45O  .

18.	 A simply supported beam AB  of span 10 m carries vertical concentrated loads of 5 kN, 10kN, 6kN ,8kN at 2m, 4m,6m and 8m from A. The beam is also subjected to a couple of 4kN.m (clockwise) at mid-span.  In addition, an uniformly distributed load of 1 kN/m acts over the entire length of the beam. The beam is provided with hinged support at A and roller support at B.  Determine the reactions at A and B.

(OR)

[P.T.O]

19.	A block of weight of 500 N just starts moving down a rough inclined plane supported by a force of 200 N acting parallel to the plane and it is at the point of moving up the plane when pulled by a force of 300 N parallel to the plane. Determine the inclination of the plane and the coefficient of friction between the inclined plane and the block.

Determine the magnitude and nature of force in each member of the truss shown in Fig. 1 using the method of joints.

12kN

15 kN

6 kN

B

C

D

4.5kN

3m

A

E

F

4m						4m

FIG. 1

(OR)

21.	Determine the mass moment of inertia of the solid cone of height `h’ and base radius ‘R’ about (i) the axis of rotation. (ii) the axis though vertex normal to the axis of rotation.

22.	A projectile is aimed at a target on the horizontal plane and falls 12 m short when the angle of projection is 15O . The projectile overshoots the target by 24 m  when the angle of projection is   45O  .   Determine the angle of projection of the projectile to hit the target accurately.

(OR)

23.	A ball is dropped from the top of a tower 30 m high. At the same instant, a second ball is thrown upward from the ground with an initial velocity of 15 m/s. Determine the time elapsed and height from the ground when the two balls cross each other. Also determine the relative velocity.

24.	A block weighing 2500 N is at rest on a horizontal plane for which the coefficient of friction is 0.20. The block is pulled by a force of 1000 N acting at an angle of 30O    to the horizontal. Determine the velocity of the block after it moves 30 m starting from rest. Also determine the distance moved by the block if the force of 1000 N is removed after travelling a distance of 30 m starting from rest. Use work energy principle.

(OR)

25.	A golf ball is dropped from a height of 10 m on a fixed steel plate. The coefficient of restitution is 0.894. Determine the height to which the ball rebounds on the first, second, third and fourth bounces.