Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	SURVEYING - I	 								              Time : 3 hours

Subject Code: 	CE243			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	In the chaining process, the chain is handled by two man called the ________and the _______.

2.	________is an error which is constant in sign and magnitude for constant field conditions.

3.	The horizontal angle between the true meridian and a line is called a true bearing of the line. What is it known as?

4.	When the vertical circle of the instrument is on the left of the observer when taking a reading the position is called the __________.

5.	It is necessary when the instrument has to be set up at more than one station. What is it?

6.	Mechanical method of three – point problem on the plane table surveying also known as ______.

7.	It is a Fixed reference point of known elevation. What is it?

8.	It is the elevation of the plane of collimation when the instrument is correctly leveled. What is known as?

9.	This is the chord of the curve joining the starting point and the end of the curve ________.

10.	Length of a vertical curve in chains connecting two gradients is given by _________

PART – B  (5 x 3 = 15 MARKS)

11.	The length of a survey line measured with a 30m chain was found to be 361.50m. Later, it was notice that the chain was 50mm too short. Find the actual length of the survey line.

12.	Differentiate between a closed traverse and an open traverse.

13.	What are the common types of level used?

14.	What do you understand by a Gale’s traverse table?

15.	How is a simple curve set out by perpendicular off sets form the long chord?

PART – C  (5 x 15 = 75 MARKS)

16.	Explain in briefly the various conventional signs to indicate various features.

(OR)

17.	Two points A and B 220m apart were selected on the near bank of a river. The bearings of a pole P fixed on the opposite bank were observed from A and B to be N 25ºE and N 50ºW, find the width of the river?

18.	The Fore bearings of the lines of a traverse are given below:

Line				Fore Bearings

AB					70º 18

BC					126º30

CD					220º16

DE					290º15

EA					330º6

Find the interior angles?

(OR)

[P.T.O]

19.	Describe the trial and error method of solving the three point problem.

20.	Describe the various errors they can occur in Levelling.

(OR)

21.	The following staff readings were taken in a leveling operation. The staff reading given in bracket was taken on the inverted staff. Enter the readings in a level book form and workout the R.L of the various points by the plane of collimation method. Take the R.L of station. A equal to 85.00m.

Staff readings				Remarks

0.950						B.S to A

1.125						I.S to B

(3.260)						I.S to C

1.805						F.S to D

1.970						B.S to D

1.205						F.S to E

22.	Explain the various rules used for determining the corrections to balance the survey.

(OR)

23.	The following traverse is run to set out a line AE 1890 m long at right angles to a given line AB:

Line	Length in m		Bearings		Lot in m		Dep in m

AB			-			360º			-			-

AC			840			119 º30		-413.6		+ 731.1

CD			1020		85 º 24		+81.81		+ 1016.0

DE			  ?			    ?				-			-

Calculate the required length and bearing of DE.

24.	Explain in briefly the classification of curves.

(OR)

25.	The length of the long chord of a simple curve of radius 400m is 100m. Find the lengths of the perpendicular offsets from the long chord at 10m intervals.