Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	SURVEYING - II									               Time : 3 hours

Subject Code: 	CE244			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is meant by the word ‘tacheometry’?

2.	Distinguish between stadia and subtense method in tacheometry.

3.	What is meant by well conditioned triangle?

4.	What is meant by GPS?

5.	What is meant by true value of quantity?

6.	What is meant by weight of an observation?

7.	What is meant by celestial sphere?

8.	What is meant by polar radius of the earth?

9.	What is meant by photogrammetry?

10.	What is meant by isocentre?

PART – B  (5 x 3 = 15 MARKS)

11.	The angle subtended by a theodolite with the two extremeties of a subtense bar is 1840. The targets of the subtense bar are 2m apart. Find the horizontal distance between subtense bar and the theodolite.

12.	What is meant by phase of signal? Deduce an expression for correction to phase of signal.

13.	Given  = 32 10 20 weight 6, find the weight of 3.

14.	Write short notes on astronomical co-ordinate systems.

15.	What are soundings? Discuss various methods of taking soundings.

PART – C  (5 x 15 = 75 MARKS)

16.	Deduce expression for horizontal distance and elevation for staff held vertical and line of sight inclined by stadia tachometry.

(OR)

17.	A tachometer was set up at station A and the following readings were obtained on a vertically held staff:

Station		Staff		Vertical		Hair readings 		Remarks

Station		 angle

B.M		-218		3.225, 3.550, 3.875	R.L of B.M = 100.000 m

A

B			+836		1.650, 2.515, 3.380

Calculate the horizontal distance from A to B and the R.L of B. K = 100, s = 0.4

[P.T.O]

18.	From a satellite station S, 60m from triangulation station C, the horizontal angles observed to other stations are as follows:

Instrument			Sight to					Horizontal angle

S					A						0000

B						715432

C						2961202

The approximate lengths of AC and BC are 18024m and 23761m respectively. Compute the angle subtended at station C by the other two stations A and B

(OR)

19.	What is a well conditional triangle? Prove that isoscles triangle is a well conditioned triangle with two angles opposite to equal sides, approximately equal to 5614.

20.	Adjust the angles of a triangle ABC from the following data.

A	=	641240	weight	3

B	=	551423	weight 	2

C	=	603321	weight 	1

(OR)

21.	a.	What is spherical excess? Discuss its determination.

b.	Explain clearly how you would adjust a geodetic quadrilateral.

22.	Derive an expression to get distance between two points on a parallel of latitude.

(OR)

23.	What do you mean by ‘equation of time’? How its value is determined.

24.	To determine the average scale of an aerial photograph, three points A, B and C were selected. Their elevations were determined from a contour map as 1400m, 1900m and 1100m. If the flying height of the aircraft above mean sea level is 3500m and the focal length of camera lens is 1600mm, calculate the average scale of the aerial photograph.

(OR)

25.	Discuss the methods of fixing horizontal and vertical control for a hydrographic survey.