Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	MECHANICS OF FLUIDS							              Time : 3 hours

Subject Code: 	CE245			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Specific volume is the ----------------------------------- of mass density.

2.	When the fluid is at rest, the shear stress is maximum. (True or False)

3.	For compressible flow, density is constant. (True or False)

4.	If the Reynolds number is less than 2000, the flow is called----------------------

5.	The condition for maximum discharge over a Broad-crested weir is equal to-------

6.	Pitot-tube is a device used for measuring the discharge of flow at any point in a Pipe (or) a channel. (True or False)

7.	The pipe of different lengths and different diameters connected end to end is called----------------.

8.	High pressure having the effect of hammering action on the walls of the pipe is called water hammer. (True or False).

9.	Dimensional analysis is the method of dimensions, in which fundamental dimensions are ---------10.	Dynamic similarity means the similarity of motion between model and prototype.

(True or False).

PART – B  (5 x 3 = 15 MARKS)

11.	What do you mean by gauge and vacuum pressure?

12.	A stream function is given by W =5x-6y, calculate the velocity components and the resultant velocity at any point.

13.	Write the advantages of triangular notch (or) weir over rectangular notch (or) weir.

14.	Find the loss of head, when a pipe of diameter 200mm is suddenly enlarged to a diameter of 400mm. The discharge through the pipe is 0.25m3/sec.

15.	Explain the term “dimensionally homogeneous equation”.

PART – C  (5 x 15 = 75 MARKS)

16.	Find the magnitude and direction of the resultant force due to water acting on a roller gate of cylindrical form of 4m diameter, when the gate is placed on the dam in such a way that, water is just going to spill. Take the length of the gate as 8m.

(OR)

17.	a. 	Explain the terms “meta-centre and meta-centric height”                               			(3)

b.	Classify the types of fluids and define each one of them                               			(6)

c.	Calculate the pressures individually due to a column of 0.3 m of water, 0.3m of an oil of specific gravity 0.8 and 0.3m of mercury of specific gravity 13.6. Take density of water 1000kg/m3.                                                                                                 			(6)

18.	a.	A 25cm diameter pipe carries oil of specific gravity 0.90 at a velocity of 3m/s. At another

section the diameter is 20cm. Find the velocity at this section and also mass rate of flow of oil.

[P.T.O]

(9)

b.	What are the assumptions made in the derivation of Bernoulli’s equation? Write the

Bernoulli;s equation for real fluid with justification.                                             		(6)

(OR)

19.	Derive Euler’s equation of motion and then Bernoulli’s equation.

20.	a.	Write the discharge formulae over a rectangular notch, triangular notch and over trapezoidal

notch.                                                                                                  					(6)

b.	A broad crested weir of 50m length has 50 cm height of water above its crest. Find the maximum discharge. Take Cd=0.60. Neglect velocity of approach. If the velocity of approach is to be taken in to consideration, find the maximum discharge when the channel has a cross-sectional area of 50m2 on the upstream side.                             			(9)

(OR)

21.	a.	Define an orifice and a mouthpiece and its classifications.                              		(9)

b.	The head of water over an orifice of diameter 40mm is 10m. Find the actual discharge and

actual velocity of the jet at vena-contracta. Take Cd=0.6 and Cv= 0.98               		(6)

22. 	a.	Three pipes of lengths 800m, 500mand 400m and of diameters 500mm, 400mm and 300mm

respectively are connected in series. These pipes are to be replaced by a single pipe of length 1700m. Find the diameter of the single pipe.                                              			(6)

b.	A pipe of diameter 20cm and length 2000m connects two reservoirs, having difference of

water level as 20m. Determine the discharge through the pipe. Take f = 0.015.  		(9)

(OR)

23. 	A pipe of diameter 300mm and length 3500m is used for the transmission of power by water. The total head at the inlet of the pipe is 500m. Find the maximum power available at the outlet of the pipe, if the value of f =0.006.

24. 	a.	How are the repeating variables selected for dimensional analysis?              			(6)

b.	What is meant by geometric, kinematics and dynamic similarities?               			(9)

(OR)

25. 	A pipe of diameter 1.5m is required to transport an oil of specific gravity 0.90 and velocity 3X10-2  poise at the rate of 3m³/sec. Tests were conducted on a 15cm diameter pipe using water at 20º C. Find the velocity and rate of flow in the model. Viscosity of water at 20° C =0.01 poise.