Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	APPLIED HYDRAULICS AND FLUID MACHINES	              Time : 3 hours

Subject Code: 	CE246			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Write Manning’s formula.

2.	Define specific energy.

3.	Define Displacement thickness.

4.	Write the mathematical Expression for Lift force?

5.	Force excreted by a jet of water on a stationery inclined plate in the direction of jet is _______.

6.	Define Impulse Momentum Principle.

7.	Specific speed of turbine is given by _________.

8.	What is meant by priming of the pump?

9.	List the three casing in Centrifugal pump.

10.	Define Hydraulic efficiency of the Turbine.

PART – B  (5 x 3 = 15 MARKS)

11.	Explain the terms:	a.	Rapidly Varied flow		b.	Gradually varied flow.

12.	Define Boundary Layer, why does it increase with distance from the upstream edge.

13.	Find the force exerted by a jet of water of diameter 60mm on a stationary plate normally with a velocity of 20m/s.

14.	What is an Air vessel? What are its functions?

15.	How are turbines classified?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Explain the various types of flow in open channels.								(10)

b.	Find the velocity of flow and rate of flow through a rectangular channel, 6m wide and 3m deep, when the bed slope as 1 in 2000. take Chezy’s constant C = 55.				(5)

(OR)

17.	a.	Find the expression for loss of head in a hydraulic jump.							(5)

b.	Find the bed slope of a trapezoidal channel of bed width 4m depth of water 3m and side slope of 2 horizontal to 3 vertical, when the discharge through the channel is 20m3/s.	(10)

18.	a.	Derive an expression for Drag and Lift pressure.									(10)

b.	A flat plate 1.5m x 1.5m moves at 50 kmph in a stationary air of density 1.15 kg/m3. If the

coefficient of drag and lift are 0.15 and 0.75. Determine the drag force and lift force.	(5)

(OR)

19.	Determine the thickness of a boundary layer at the trailing edge of a smooth plate of length 4m and width 1.5m, when the plate is moving with a velocity of 4m/s in stationary air. Take kinematic viscosity of air as 1x10-5 m2/s and also determine the total drag on one side of the plate assuming the boundary layer is laminar over entire length of the plate. Take density of air      1.226 kg/m3.

[P.T.O]

20.	a.	Derive an expression for the force exerted by a jet on a stationary inclined flat plate.	(8)

b.	A jet of water of diameter 50mm moving with a velocity of 40m/s, strikes a curved fixed

symmetrical plate at the centre, find the force excretes by the jet, if the jet is deflected through an angle of 120º at the outlet of the curved plate.							(7)

(OR)

21.	a.	Derive an expression for the force exerted by a jet on a flat vertical flat plate moving in the

direction of the jet.															(7)

b.	A jet of water of diameter 7.5 cm strikes a curved plate at its centre with a velocity of 20m/s.

the curved plate is moving the velocity of 8 m/s in the direction of jet. The jet is deflected through an angle of 165º. Assuming the plate smooth find.							(8)

i.	Force exerted on the plate in the direction of jet.

ii.	Power of the jet.

22.	a.	Sketch the velocity triangles at inlet and outlet of pelton wheel.					(5)

b.	A pelton wheel has to be designed for the following data. Power to be developed = 6000kW.

Net head available = 300m, speed = 550rpm, Ratio of jet diameter to wheel diameter = 1/10 and the overall efficiency = 85%. Find the no. of jets, diameter of jets and diameter of the wheel and the quantity of water required.										(10)

(OR)

23.	a.	Derive an expression for the efficiency of the draft tube.							(5)

b.	Calculate the diameter and speed of the runner of a Kaplan turbine developing 6000 kW

under an effective head of 5m. Overall efficiency of the turbine is 90%. The diameter of the boss is 0.4 times the external diameter of the runner. The turbine speed ratio is 2.0 and the flow ratio is 0.6.																(10)

24.	a.	Define Manometric efficiency of a centrifugal pump.								(5)

b.	The impellor of a centrifugal pump has an external diameter of 450mm and the internal

diameter of 200mm and it runs at a speed of 1440 rpm. Assuming a constant radial flow through the impellor at 2.5 m/s and that the vanes exit are set back at an angle of 25º. Determine 																	(10)
i.	Inlet vane angle

ii.	The angle, absolute velocity of water at exit makes with the tangent

iii.	Work done per N of water

(OR)

25.	a.	Describe the effect of acceleration of piston on a velocity and pressure in the suction and

delivery pipes.																(8)

b.	A single acting reciprocating pump has a cylinder of 0.25m diameter. The stroke length is 0.35m. Speed of the crank is 60 rpm. It delivers 0.017 m3/s. Find the coefficient of Discharge and Percentage slip of the pump.