Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	MECHANICS OF SOILS							              Time : 3 hours

Subject Code: 	CE247			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Ratio of unconfined compressive strength of an undisturbed specimen of soil to the unconfined compressive strength of a specimen of the same soil after remolding at unaltered water content is ----------------

2.	If uniformity coefficient of a soil is twice its coefficient of curvature, find the ratio between D60 & D30.

3.	The discharge and seepage velocities of a soil are 1 x10-3 cm/s & 2 x10-3 cm/s. find the void ratio of the soil.

4.	The capillary water fills all the pores in the soil, to a certain distance above the water table, this distance is known as ----------

5.	Square root of time method is used to find --------------------

6.	Work out theoretical maximum dry density for a soil sample having specific gravity 2.7 and optimum moisture content 16%. Find dry density of the soil.

7.	Write the assumption given by Boussineque’s stress distribution.

8.	A curve or contour connecting all points below ground level of same vertical pressure is called ---------

9.	The UCC strength of saturated clay 200kPa. If a cell pressure of 100kPa is maintained on a similar specimen of clay in UCC untrained condition, find deviator stress at which the sample will fail.

10.	If a slope represents the boundary surface of semi infinite soil mass, and the soil properties for all identical depths below the surface are constant is a --------------

PART – B  (5 x 3 = 15 MARKS)

11.	Darw the phase diagram for soil.

12.	What are the steps in the construction of a flow net?

13. 	What is primary consolidation settlement?

14. 	Compare Boussineque’s and Westerguard’s solutions.

15. 	What are the types of failure of finite slopes? Explain with sketch.

PART – C  (5 x 15 = 75 MARKS)

16.	The unit weight of a sand backfill was determined by field measurements to be 1746 kg per cu.m. The water content at the time of test was 8.6% and the unit weight of the solid constituents was 2.6gm per cu.cm. In the laboratory, the void ratios in the loosest and densest states were found to be 0.642 and 0.462 respectively. What was the relative density of the fill? Write the importance of this term.

(OR)

17.	The value of liquid limit, plastic limit and shrinkage limit of a soil were reported as

follows:

wL = 60% ; wP = 30% ; wS = 20%

If a sample of this soil at liquid limit has a volume of 40cc and its volume measured at shrinkage limit was 23.5 cc, determine the specific gravity of the solids. What is its shrinkage ratio?

18.	a.	i.	A horizontal stratified soil deposit consists of three uniform layer of thickness

6.4 and 12m respectively	the permeability of these layers are 8 x 10-4 cm /s,

52 x 10-4 cm /s and 6 x 10-4 cm /s, find the effective average permeability of the deposit in the horizontal and vertical direction.											(10)

[P.T.O]

ii.	Write the equation for k and T using pumping out test for unconfined aquifer.		(5)

(OR)

19.	Explain in detail the application of flow net.

20.	A building column has a footing area of 2m x 3m and transmits a pressure increment of

150 kN/m2 at its base embedded 1.6m below ground level, as shown in fig. Assuming a pressure distribution of 2 vertical to 1 horizontal, determine the consolidation settlement at the middle of clay layer. Consider the pressure variation across the thickness of clay layer also. Given the following data

i.	for sand  = 16.5 kN/m3 and sat= 18.5 kN/m3

ii.	for clay  sat= 16 kN/m3 : eo = 0.95 : Cc = 0.26

(OR)

21.	In a consolidation test, the void ratio of the specimen which was 1.068 under the

effective pressure of 214 kN/m2 , changed to 0.994 when the pressure was increased to

29 kN/m2. Calculate the coefficient of compressibility, compression index, and the coefficient of volume compressibility. Find the settlement of foundation resting on above type of clay, if thickness of layer is 8m and the increase in pressure is 10 kN/m2.

22.	A concentrated load of 40 kN acts on the surface of a soil. Determine the vertical stress increment at points directly beneath the load up to a depth of 10m and draw a lot. Also plot the variation of vertical stress increment due to load on horizontal planes at depth of 1m, 2m and 3m up to a horizontal distance of 3m on either side of center.

(OR)

23.	Explain in detail the construction of New mark’s chart with an influence value of 0.005.

24.	Explain the friction circle method of slices for stability analysis of slopes. How can  steady seepage be accounted in this method?

(OR)

25.	Laboratory results on a soil have shown that its unconfined compressive strength is   1.2 kg /cm2. In a triaxial compression test, a specimen of the soil when subjected to a chamber pressure of 0.4 kg /cm2 failed at an additional stress of 1.6 kg /cm2. Estimate the shearing strength of the same soil along a horizontal plane at a depth of 4m in a deposit. The ground water table is at depth of 2.5m from the ground level. Taking dry unit weight of soil as 1.7 g /cc and specific gravity as 2.7.