Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	REINFORCED CONCRETE STRUCTURES – I		              Time : 3 hours

Subject Code: 	CE249			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	List the different methods of design of reinforced concrete members which are accepted in practice.

2.	What is the expression for modular ratio (m).

3.	Draw the stress-strain curve at ultimate limit state of collapse in flexure for a balanced section.

4.	What is the nominal shear stress for a T-section.

5.	Write down the expression for the flexural strength of a balanced beam section for Fe 415 grade steel.

6.	What is short column and a long column.

7.	Write down the expression for the ultimate axial load capacity of a short column.

8.	While considering the design of a isolated footings, critical section for bending moment and one way shear.

9.	The effective width of T-beam is the least of the following _____________.

10.	Maximum shear strength of concrete for M20 grade is _________.

PART – B  (5 x 3 = 15 MARKS)

11.	Explain under, over reinforced and balanced section.

12.	Derive an expression for the ultimate flexural strength of a beam section.

13.	Design principle of a short column with an expression.

14.	Explain the design principle of a T-beam for flexure.

15.	Explain the design principle of a two way slab.

PART – C  (5 x 15 = 75 MARKS)

16.	A doubly reinforced concrete section has a width of 300mm and is reinforced with tension steel of area 2455 mm2 at an effective depth of 600mm compression steel area of 982 mm2 is provided at an effective cover of 60mm. Use fck = 20 N/mm2  and fy = 415 N/mm2 Estimate the ultimate moment capacity of the section. Using the stress – strain curve of steel.

(OR)

17.	Determine the area of tensile reinforcement required for a singly reinforced beam section of size 300 x 550mm effective to carry a factored moment of 175 kNM use fck = 20 N/mm2 and

fy = 415 N/mm2.

18.	Determine the limiting moment for a R.C section of size 250 x 550 mm reinforced on tension side with 4 Nos – 20mm  bars. use fck = 20 N/mm2 and fy = 415 N/mm2.

(OR)

19.	Determine the shear reinforcement required for a reinforced concrete beam of T-section with a flange width (bf) = 1500mm flange depth (Df) = 100mm, a web width (b) = 300mm and an effective depth of 500mm. The beam is reinforced with 6 Nos – 25mm  on tension side and       4 Nos – 14mm  on compression sick. The beam supports a uniformly distributed factored load of 75 kN/m over a span of 9m. Use fck = 20 N/mm2 and fy = 415 N/mm2.

[P.T.O]

20.	A reinforced concrete column of un supported length of 3.0 m is to be designed for a factored axial load of 2500 kN. Determine the cross-sectional dimensions of the column and reinforcement required when there is no restriction on the cross-sectional dimensions.

Fck = 20 N/mm2			and		fy = 415/mm2

(OR)

21.	Determine the reinforcement of grade Fe415 steel required for a square column of size 400 x 400mm subjected to a factored direct load and factored moment of 2000 kN and 250 kNM, respectively, when the reinforcement is to be placed on two sides (parallel) with a nominal cover of 30mm. Use fck  = 20 N/mm2  and fy =415 N/mm2. Use column interaction curve

22.	A reinforced concrete beam having a rectangular section 300mm wide with 2 Nos of 12 mm  at an effective depth of 550mm. The section is subjected to a service moment of 40 kNm.

Adopt fy = 415 N/mm2, fck = 20 N/mm2.

(OR)

23.	A reinforced concrete beam of span 5m has rectangular section 250mm wide by 500mm depth. The beam is reinforced with 3 Nos of 16mm  on the tension side at an effective depth of 450mm and 2Nos of 16mm  on the compression side at a cover of 50mm from compression face. Assume fck = 15 N/mm2 and fy = 250 N/mm2. Estimate the maximum service live load on the beam.

24.	A simply supported R.C slab having an over all depth of 150mm is reinforced with 12mm  bars at an effective depth of 130mm. The spacing of bars is 100mm. The effective span of slab = 4m. If the self wt of slab and finishes is 4.2 kN/m2, estimate the maximum permissible live load on the slab. Adopt M – 15 grade concrete and Fe 250 grade steel.

(OR)

25.	Design a suitable R.C.C column to support an axial load of 800 kN. Adopt M-20 grade concrete and Fe 415 grade steel.