Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	REINFORCED CONCRETE STRUCTURES - II		              Time : 3 hours

Subject Code: 	CE250			         						                Maximum Marks: 100

Use of IS: 3370, IS: 456 and IS:1893 codes may be permitted

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	For the economic utilization of available space, the ____________type of staircase is recommended.

2.	What is a shear key?

3.	For an underground water tank, the active earth pressure varies _____________ along the depth of the tank.

4.	What type of thrust does a conical dome in a circular tank is subjected to?

5.	When a solid slab bridge is recommended?

6.	For IRC Class AA loading, the nose to tail spacing between two successive vehicles shall not be less than _________ m.

7.	What is ductility in reinforced concrete members?

8.	One of the methods used for analyzing wind loads is ________________.

9.	For complete yield line pattern to develop, the slab must be ____________ reinforced.

10.	Write the expression for the ultimate moment along the yield line for a square slab simply supported and supporting uniformly distributed load.

PART – B  (5 x 3 = 15 MARKS)

11.	Why counterforts are provided in retaining walls? Draw a counterfort retaining wall and mark its components.

12.	Briefly explain with a neat sketch, (i) Contraction joint and (ii) Expansion joint. List the

commonly used joint sealing compounds.

13.	What provision is made in an RC bridge against impact effect for class A or Class B loading? Write the formula to determine the impact factor.

14.	What is a substitute frame? Give an example.

15.	List the characteristic features of yield lines.

PART – C  (5 x 15 = 75 MARKS)

16.	The details of a doglegged staircase of a multi-storied building are given below:

Number of steps in the going = 10

Tread, T = 300 mm, Rise, R = 150 mm

Width of landing slab on either side = 1m

M-20 grade concrete and Fe-415 HYSD bars.

Design and detail any one of the flights of the staircase.

(OR)

17.	A cantilever retaining wall to retain earth embankment is 4m above ground level. The density of earth is 18 kN/m3 and its angle of repose is 30. The embankment is horizontal at top. The safe bearing capacity of the soil may be taken as 200 kN/m2 and the coefficient of friction between soil and concrete is 0.5. Using M-20 grade concrete and Fe-415 HYSD bars, design and detail the retaining wall.

18.	A rectangular water tank placed on ground is of size 6m x 2m and a maximum depth of

2.5 m. Design the side walls of the tank using M-20 grade concrete and Fe-415 HYSD

bars. Sketch the details of reinforcement neatly.

(OR)

[P.T.O]

19.	An R.C.C. circular water tank, which is resting on ground with a flexible base and spherical dome for storing 500,000 litres of water. The depth of the storage is to be 4m. Free board = 200 mm. Using M-20 grade concrete and Fe-415 HYSD bars, design the tank. The permissible stresses should comply with the values recommended in IS: 3370 and IS: 456-2000 codes. Sketch the details of reinforcements in the dome and the tank walls.

20.	The details of a solid slab bridge is given below:

Clear span						: 4.5 m

Clear width of roadways			: 7 m

Average thickness of wearing coat	: 80 mm

Unit weight of concrete				: 24000 N/m3

Design the bridge for class A loading adopting M 20 grade concrete.

(OR)

21.	a.	Explain Courbon’s method of load distribution in interconnected girders. When this

method is applicable? 				    												(7)

b.	With a neat sketch explain the components of a T-beam bridge. Briefly explain how the

main components are designed.				    										(8)

22.	Analyse the building frame shown in fig. 1 subjected to horizontal forces by Portal  method.

(OR)

23.	a.	What are the assumptions made in the earthquake resistant design of structures?      		(3)

b.	Discuss briefly the various codal provisions for the dynamic analysis of a building as

per IS : 1893-2002. 			    												(12)

24.	A rectangular slab is of size 6 m x 4 m and simply supported at the edges for a service load of

4 kN/m2. Assume coefficient of orthotrophy () as 0.7, M-20 grade concrete and   Fe-415 HYSD bars. Design the slab.

(OR)

25.	Design a circular slab of diameter 5m, which is simply supported at the edges. Let live load = 4 kN/m2. 	Adopt M-20 grade concrete and Fe-45 HYSD bars. Assume load factors according to IS: 456-2000.