Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	DESIGN AND DRAWING (RCC & STEEL)			              Time : 4 hours

Subject Code: 	CE251			         						                Maximum Marks: 100

Use of relevant IS codes is permitted

#### Answer ALL questions

PART – A  (1 x 55 = 55 MARKS)

1.	Design a cantilever retaining wall to retain an earth embankment with a horizontal top 4 m above ground level. Density of earth =18 kN/m3. Angle of internal friction = 300. SBC of soil = 200 kN/m2. Coefficient of friction between soil and concrete = 0.5. Adopt M20 Grade concrete and Fe 415 HYSD bars.

Draw the following views:

a.	Cross section of retaining wall showing reinforcement details.

b.	Longitudinal section showing curtailment

(OR)

2. 	Design an RC circular water tank resting on the ground with a flexible base and a spherical dome for a capacity of 5,00,000 litres. The depth of storage is to be 4 m. Free board= 200 mm. Use M 20 grade concrete and Fe 250 grade steel.

Draw the following views:

a.	Cross section of the tank showing reinforcement details in dome, tank walls and floor slab.

b.	Plan of the tank showing reinforcement details.

PART – B  (1 x 45 = 45 MARKS)

3. 	Design a rectangular steel water tank to store 130 KL of water.

Height of staging 						=  12 m

SBC of soil							=  250 kN/m2

Intensity of wind pressure				=  1 kN/m2

Permissible bearing stress on concrete	=  4 N/mm2

Draw the elevation and plan showing the details of the tank.

(OR)

4. 	Design a gantry girder to carry an electric overhead traveling crane for an industrial shed to suit the following data:

Crane – load lifting capacity				=  200 kN

Weight of crane excluding trolley			=  150 kN

Weight of trolley							=   75 kN

Distance between centres of gantry girder 	=   20 m

Minimum approach of crane hook			=  1.2 m

Distance between centres of crane wheels	=   3 m

Span of gantry girder						=   6 m

Weight of rail section 						=   0.3 kN/m

Height of rail section						=   75 mm

Draw a cross section of the crane girder to a suitable scale.