Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	DESIGN OF STEEL STRUCTURES					              Time : 3 hours

Subject Code: 	CE252			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	A tension member, if subjected to possible reversal of stress due to wind, the slenderness ratio of the member should not exceed __________

2.	Effective length of column effectively held in position at both ends and restrained in direction at one ends, is _____________

3.	For a simply supported beams, the maximum permitted deflection is _________

4.	The connection of a beam to another beam by means of an angle at the bottom and an angle at the top, is known as __________

5.	The depth of plate girder is adopted as ___ to ____ of the span of girder.

6.	The joint between flange and web is subjected to shear stress. The shear stress and shear flow on the joint is given by __________

7.	The effective length of a single angle member in welded truss is equal to ______

8.	Gantry girder has to be designed to resist __________

9.	The two processes for manufacturing or forming the light gauge section are _____ and ________

10.	Give some common shapes of cold formed light gauge sections.

PART – B  (5 x 3 = 15 MARKS)

11.	Draw the section of a gusseted base.

12.	What is meant by curtailment of plate?

13.	Mention the elements of a plate girder.

14.	What is meant by Gable frame?

15.	What is meant by Flat width ratio?

PART – C  (5 x 15 = 75 MARKS)

16.	Long leg of an ISA 150mm x 75 mm is connected to a gusset plate by 20 mm diameter rivets in two rows. The gauge space is 55mm and the staggered pitch is 40mm. Determine the thickness of the angle which would be sufficient to transmit a pull of 250 kN. Allowable tensile stress is 0.6fy.

(OR)

17.	A column 5m long is to support a load 4500 kN. The ends of the column are effectively held in position and direction. Design the column if rolled steel beams and 18 mm plates are only available.

18.	A hall of clear dimensions 15m x 6m is to be covered by RCC slab flooring 12 cm thick resting over RS joist spaced at an interval of 3m center to center. Terrazzo finishing 2 cm thick is to be provided over the RCC slab. The live load on the slab is 4 kN / sq.m. Joist are resting over 30 cm thick wall. Design the floor joist if the permissible stresses in bending and shear are 165 MPa and 100 MPa respectively. The unit weight of RCC and finishing is 24 kN / Cu.m.

(OR)

[P.T.O]

19.	Design a suitable section for a beam girder carrying uniformly distributed load of 55 kN / m including self weight over an effective span of 15.6 m.

20.	A plate girder has the following elements: Top and bottom plates = 480 x 28 mm each and web plate =1600 x 8 mm. Design stiffeners.

(OR)

21.	A MB 500 @ 0.869 kN / m transmits an end reaction of 130 kN to the flange of column HB 250 @ 0.510 kN/m. Design an unstiffened welded seat connection.

22.	Design an I – Section purlin to support galvanized corrugated iron sheet roof. The purlins are 1.25m apart over a roof trusses spaced 5 m center to center. The roof surface has an inclination of 300 to the horizontal. The weight of iron sheet is 0.1331 kN / sq.m. The weight of fixtures etc., 0.053 kN / sq.m. The design wind pressure for medium permeability is 1.20 kN / sq.m. parallel to ridge.

(OR)

23.	Explain the step by step procedure followed in the design of Gantry Girder.

24.	Find the column section properties and allowable load for column section shown in fig.1. The effective length of column is 3.2 m. Take fy = 235 N / Sq.mm

92mm

4

4

4

92

4

Fig.1

(OR)

25.	Determine the allowable load per meter run on a beam with hat section shown in fig.2. The effective span of beam is 3m. Steel has a yield point of 235 N / sq.mm and

E = 2 x 105 N / sq.mm.

120mm

2 mm

5mm

5mm

Fig.2