Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	SANITARY ENGINEERING						              Time : 3 hours

Subject Code: 	CE253			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The Wet Weather flow [WWF] is also known as ______________

2.	If Ultimate BOD is 300 mg/l then the BOD5 at 200c would be _____________

3.	The first unit operation in waste water treatment plants is ____________

4.	In the Grit chambers design a detention period of ________is usually adopted

5.	Find the length of the sedimentation tank if the detention time is 4 hours and the flow through velocity in the tank is 5m/Hour.

6.	Psychoda  fly nuisance available in___________

7.	A Physical washing operation of sludge is know as __________

8.	The minimum amount of dissolved oxygen that should available in the river for effective survival of fish is ___________

9.	In traps ___________prevents the passage of foul air or gas through it, and allows the sewage to pass through it

10.	A _____________ is a pipe through which human excreta flows

PART – B  (5 x 3 = 15 MARKS)

11.	Differentiate sewage and sullage

12.	What do you understand by the terms self cleansing velocity and limiting velocity in sewers?

13.	Define the term SLOUGHING with neat diagram

14.	The moisture content of sludge is reduced from 95% to 90% find the decrease in the volume of sludge

15.	List the classifications of traps, based on shape and use.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Differentiate clearly between BOD,COD,TOD and ThOD 							(8)

b.	The 5 day BOD at 200c of a sewage sample was found to be 100 mg/l. Calculate 2 day BOD

at 300c for the same sample.

Assume K D(20)  = 0.2 day-1. Take K D(T) = K D(20) {1.047}T-20    						(7)

(OR)

17.	a.	What do you understand by ‘sewer appurtenances’? Why they are necessary? List out

various appurtenances that are commonly used.                                                         (3+3+4)

b.	Discuss the various points that should kept in mind while selecting the site for a sewage

pumping station 																(5)

18.	a.	What do you understand by grit chambers? Why it is necessary to provide grit chamber?

Explain the configuration of a grit chamber with the help of neat sketches

(OR)

[P.T.O]

19.	a.	What do you understand by sedimentation of sewage? Describe in brief various types of

settling.																		(10)

b.	Why depth of sedimentation a tank is not important while designing when compared to plan area of the sedimentation tank?													(5)

20.	a.	What is meant by Activated sludge process? Describe with sketches the treatment of sewage

by ASP. Mention the advantages and disadvantages of this system

(OR)

21.	Design a septic tank for the following data with SOAK well arrangement of effluent disposal.

(i)	No. of people 						= 100

(ii) 	Sewage / Capita day  					=120 liters

(iii) 	Desludging period 					=  2 year

(iv) 	Length to width ratio					= 4 : 1

(v) 	Perculation capacity of filtering media in soakwell		= 1250 Lts /m3/day

(vi)	Depth of the soak well 				= 2 m

(vii) Rat of sludge deposit 					=30 lts/capita/year

(viii)Depth of septic tank					=1.5 m

22.	Discuss the anerobic digestion of sludge under the following heads.

a.	Definition and composition of sludge

b.	Fundamental mechanism of anaerobic digestion

c.	End product and their disposal

(OR)

23.	a.	Write a detailed note on self purification of stream and the factors affecting self

purification of streams. 													(9)

b.	Describe sewage farming and sewage sickness								(6)

24.	a.	Critically discuss one and two pipe system of plumbing 						(9)

b.	List out the characteristics of a good water closet								(6)

(OR)

25.	Write short notes on:															(5+5+5)		a.	Physical,chemical,Biological characteristics of sewage

b.	Rural sanitation and its importance

c.	Reuse of treated sewage