Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	DESIGN AND DRAWING

(IRRIGATION AND ENVIRONMENTAL ENGINEERING)   Time : 4 hours

Subject Code: 	CE256			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (1 x 55 = 55 MARKS)

1.	The following details refer to the particulars of a canal to be taken across a drain.

Design a siphon aqueduct.													(15)

a.	Canal

Bed width = 15m

Bed level = +25.00m

Depth at Full supply level = 2m

Discharge = 20m3/s

Side slope = 1 ½ : 1

b.	Drain

Bed width = 25m

Bed level = +23.20m

Maximum flood level = +25.20m

Discharge = 70m3/s

Side slope = 1 ½ : 1

General ground level = +25.00m

Maximum allowable velocity in the canal = 1.5m/s

Maximum allowable velocity in the drain = 2.5m/s

Good foundation soil is available at +22.5m

Design and draw the following:

i.	Half plant at top and half plan at foundation								(20)

ii.	Longitudinal section													(20)

(OR)

2.	Design a Notch type canal drop for a depth of 2m with the following particulars. Discharge is 4m3/s. Full supply depth is 1.5m. Half supply depth is 1.0m.					(15)

Data Particulars			Upstream side		Downstream side

Bed width				+6.00m				+6.00m

Full supply level			+11.5m				+9.5m

Bed level				+10.0m				+8.00m

The slope protection may be provided by a stone revetment of 50cm thick over gravel. The flooring for water cushion is concrete. The good foundation soil is available at 8.50m.

Draw the following views.

a.	Longitudinal section through the centre line of the canal drop.				(20)

b.	Plan half at the top and half at foundation level.							(20)

[P.T.O]

PART – B  (1 x 45 = 45 MARKS)

3.	Design a rapid sand filter unit for 4.5 million litres/day of supply with all its principal components.																(15)

Draw the following views:

a.	Plan of the filter unit.												(15)

b.	Longitudinal section of the unit.									(15)

(OR)

4.	Design suitable dimensions of a circular trickling filter units for treating 4 million litres of sewage per day. The BOD of sewage is 150 mg/l. Also design suitable dimensions of its rotary distribution system as well as drainage system.								(15)

Draw the following views:

a.	Plan of the filter unit													(15)

b.	Cross-section of the unit.												(15)