Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	MECHANICS OF FLUIDS							              Time : 3 hours

Subject Code: 	CE277			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the effect when an ideal liquid is subjected to shear stress?

2.	The zero of mechanical pressure gauges represents _____________.

3.	If the stream function (ψ) satisfies Laplace equation, the flow is a possible case of___________.

4.	What is an equipotential line?

5.	Write one advantage of venturimeter over orificemeter .

6.	Weir is a _____________of a notch.

7.	Friction factor is a function of _____________

8.	What is the condition for maximum transmission of power through pipes?

9.	Write the dimension of kinematic viscosity.

10.	Weber’s number relates two forces.  Name them.

PART – B  (5 x 3 = 15 MARKS)

11.	A isosceles triangular plate of base width 1.5m and height 2m lies immersed in water with the apex downwards.  The base of the plate is horizontal and coincides with the free surface of water.  Calculate the total pressure and centre of pressure.

12. 	A 0.6m diameter pipe line carries oil of specific gravity 0.85 at 82500m3 per day.  The friction head loss is 8.5m of liquid per 1000 m of pipe run.  It is planned to place pumping station at every 20 km along the pipe.  Make calculations for the pressure drop in kN/m3 between the pumping stations.

13. 	Name the classification of orifice.

14. 	Three pipes of lengths 800m, 500m and 400m and of diameters 500mm, 400mm and 300mm respectively are connected in series.  These pipes are to be replaced by a single pipe of length of 1700m.  Find the diameter of the single pipe.

15. 	In 1 in 40 model of a spillway, the velocity is 2m/s.  Find the corresponding velocity in the prototype.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	The pressure inside a tube of a wheel as shown by mechanical pressure gauge is

28 x 104 N/m2.  What is the absolute pressure?											(3)

b.	Find the magnitude and direction of the resultant force due to water acting on a roller gate of cylindrical form of 4.0 m diameter, when the gate is placed on the dam in such a way that water is just going to spill.  Take the length of the gate as 8m.								              (12)

(OR)

17.	A trapezoidal channel 2 m wide at the bottom and 1 m deep has side slopes 1:1.

Determine:

- the total pressure, and

- the centre of pressure on the vertical gate closing the channel when it is full of water.

18.	a.	A 0.30 m diameter pipe, conveying water, branches into two pipes of diameters 0.20 m and 0.15m respectively.  If the average velocity in the 30 cm diameter pipe is 2.5 m/s, find the discharge in this pipe.  Also determine the velocity in 0.15 m pipe if the average velocity in 0.20 m diameter pipe is 2 m/s.	(7)

[P.T.O]

b.	A jet of water from a 25 mm diameter nozzle is directed vertically upwards.  Assuming that the jet remains circular and neglecting any loss of energy, what will be the diameter at a point 4.5 m above the nozzle, if the velocity with which the jet leaves the nozzle is 12 m/s. 						  (8)

(OR)

19.	a.	A stream function is given by Ψ=5x−6y.

Calculate the velocity components and also magnitude and direction of the resultant velocity at any point.																			(7)

b.	A vessel, cylindrical in shape and closed at the top and bottom, contains water upto a height of 0.80m.  The diameter of the vessel is 0.20 m and length of vessel 1.20 m.  The vessel is rotated at a speed of 400 r.p.m. about its vertical axis.  Find the height of paraboloid formed.		                (8)

20.	The inlet and throat diameters of a horizontal venturimeter are 0.30 m and 0.10 m respectively.  The 	liquid flowing through the meter is water.  The pressure intensity at inlet is 13.734 x104 N/m2 while the 	vacuum pressure head at the throat is 0.37 m of mercury.  Find the rate of flow.  Assume that 4% of the 	differential head is lost between the inlet and throat.  Find also the value of Cd for the venturimeter.

(OR)

21.	a.	The head of water over a rectangular notch is 900 mm.  The discharge is 300 litres/s.  Find the length 		of the notch when Cd = 0.62.													               (5)

b.	Water flows through a triangular right-angled weir first and then over a rectangular weir of 1 m 	width.  The discharge co-efficients of the triangular and rectangular weirs are 0.6 and 0.7 	respectively.  If the depth of water over the triangular weir is 360 mm, find the depth of water over 	the rectangular weir.														              (10)

22.	a.	An oil of sp. gr. 0.7 is flowing through a pipe of diameter 300 mm at the rate of 500 litres/s.

Find the head lost due to friction and power required to maintain the flow for a length of 1000 m.  Take Kinematic Viscosity of the oil is 0.29x10-4 m2/s.								              (8)

b.	A 150 mm diameter pipe reduces in diameter abruptly to 100 mm diameter.  If the pipe carries water at 30 liters per second, calculate the pressure loss across the contraction.  Take the co-efficient of contraction as 0.6.															              (7)

(OR)

23.	A horizontal pipe line 40 m long is connected to a water tank at one end and discharges freely into the atmosphere at the other end.  For the first 25 m of its length from the tank, the pipe is 150 mm diameter and its diameter is suddenly enlarged to 300 mm.  The height of water level in the tank is 8 m above the centre of the pipe.  Considering all losses of head which occur, determine the rate of flow.  Take f=0.01 for both sections of the pipe.

24.	A partially submerged body is towed in water.  The resistance R to its motion depends on the density ρ, the viscosity μ of water, length lof the body, velocity v of the body and the acceleration due to gravity g.  Show that the resistance to the motion can be expressed in the formR=ρL2V2ΦμρVL, lgV2

(OR)

25.	a.	A 2.5 m ship model was tested in fresh water (ρ=1000 kg/m3) and measurements indicated that 			there was resistance of 45 N when the model was moved at 2 m/s.  Work out the velocity of 40m 			prototype.  Also calculate the force required to drive the prototype at this speed through sea water 		(ρ=1025 kg/m3).															       (7)

b.	A pipe line of diameter 1.5 m is required to transport an oil of sp. gr 0.90 and viscosity 3x10-3 Ns/m2 	at the rate of 3000liters/s. Tests were conducted on a 0.15 m diameter pipe using water at 200C.find 	the velocity and rate of flow in the model .Viscosity of water at 200C =1x10-3Ns/m2.		       (8)