Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	SURVEYING										              Time : 3 hours

Subject Code: 	CE280			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define Geodetic Surveying

2.	Define differential leveling.

3.	Define Transit of Theodolite.

4.	Define a Closed traverse.

5.	Define Tachometry.

6.	Subtense bar is used for ____________

7.	Define Transition curves

8.	Define vertical curve

9.	What is base line?

10.	Define well-conditioned triangle.

PART – B  (5 x 3 = 15 MARKS)

11.	Explain the terms i) Profile Levelling ii) Fly leveling, when you do adopt these methods.

12.	Explain the following terms: Swinging & Centering of Thedolite.

13.	What are the various methods of plane table surveying?

14.	What are the advantages of transition curves?

15.	Explain the classification of triangulation system.

PART – C  (5 x 15 = 75 MARKS)

16.	The following readings have been taken from page of an old level book. It is required to reconstruct the page. Fill up the missing quantities and apply the usual checks.

| Station | B.S. | I.S | F.S | RISE | FALL | R.L | REMARKS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A | 3.125 |  |  |  |  | ? | B.M |
| B | ? |  | ? | 1.325 |  | 125.505 | T.P |
| C |  | 2.320 |  |  | 0.055 |  |  |
| D |  | ? |  |  |  | 125.850 |  |
| E | ? |  | 2.615 |  |  |  | T.P |
| F | 1.620 |  | 3.205 |  | 2.165 |  | T.P |
| G |  | 3.625 |  |  |  |  |  |
| H |  |  | ? |  |  | 123.090 | T.B.M |

(OR)

17.	a.	The following perpendicular offsets were taken from a chain line to a hedge:

| Chaniage(m) | 0 | 15 | 30 | 45 | 60 | 70 | 80 | 100 | 120 | 140 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Offsets (m) | 7.6 | 8.5 | 10.7 | 12.8 | 10.6 | 9.5 | 8.3 | 7.9 | 6.4 | 4.4 |

Calculate the area between the survey line and hedge and the offsets by (i) Trapezoidal Rule

ii) Simpson’s rule.                                                                                      		(10)

[P.T.O]

b.	What is Simpson Rule? Derive an expression for it.                                              	(5)

18.	a.	Explain with neat sketch, step by step procedure for determining the height of an electric

pole, whose base is accessible, using thedolite                                                     	(10)

b.	Define: i) Line of collimination ii) Face left observation iii) Face Right observation

iv) Transiting of Theodolite  v) The axis of Level tube                                	(5)

(OR)

19.	Explain clearly with the help of illustrations, how a traverse is balanced by three methods.

20.	a.	State three point problem in plane table surveying? Explain Bessel’s graphical solution for

the same.                                                                                                               	(9)

b.	Explain the advantages and disadvantages of plane table surveying.                    	(6)

(OR)

21.	A leveling staff is held vertical at a distance of 100m and 300 m from the axis of a tacheometer and the staff intercept for horizontal sights are 0.95m and 3.00m respectively find out the constants of the instrument. The instrument is set up station A and the staff is held vertical at a point B. With the telescopic inclined at an angle of depression of 10o to the  horizontal the readings on the staff are 2.670, 1.835 & 1.00 m. Calculate the R.L of B and its  horizontal distance from A. The H.I  is 1.42 m and R.L is 450.50 m.

22.	a.	What do you mean by an ideal transition curve? Derive the intrinsic equation

for the same.                                                                                                        	(10)

b.	What are the different methods of designating a curve? Derive a relationship between

radius and degree of curve.                                                                                    	(5)

(OR)

23.	A road bend which deflects 80o  to be designed for a maximum speed of 100km per hour, a Maximum centrifugal ration of ¼ and a maximum rate to the change of  acceleration of

30 cm/sec 3  , the curve consisting of a circular arc combined with two cubic spirals. Calculate

a) The radius of the circular arc b) The requisite length of transistion. c) The total length of

The composite curve, and d) the chainages of the beginning and end of transition curve and

Of  the junctions of the transition curves with the circular arc if the chainage of the  P.I is

42862m.

24.	Explain various tape corrections to be made in measurement of Base line.

(OR)

25.	A Nominal distance of 30m was set out with a 30m steel tape from a mark on the top of one peg to a mark on the top of another, the tape being in catenary under a pull of 100 kN and at mean temperature of 700 F. The top of one peg was 0.25 m below the top of the other. The top of the higher peg was 460m above mean sea level. Calculate the exact horizontal distance between the marks on the two pegs and reduce it to mean sea level, if the tape is standardized at a temperature of 600 F in catenary under a pull of  a) 80N b) 120N c) 100N.