Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	APPLIED ELASTICITY AND PLASTICITY			              Time : 3 hours

Subject Code: 	CE302			         						                Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	The state of stress at a particular point relative to the xyz coordinate system is given by the stress matrix

5   	  10	    -10

10	  10	      0		MPa

-10	   0	     40

Determine the normal stress and the magnitude and direction of the shear stress on a    surface in intersecting the point and parallel to the plane given by the equation.

2x – y + 3z  = 9

(OR)

2.   	Calculate the principle stresses for the stress components given below:

100		200	     100

200		-200    -300	    MPa

100		-300     400

3.   Prove that the following are Airy’s stress functions and examine the stress distribution

represented by theta:

(a)  Ф = Ax2 + By2 	(b)  Ф = Ax3	     (c)  Ф = A (x4 – 3x2y2)

(OR)

4.   Derive an expression for determining airy polynomial stress function for cantilever having

uniformly distributed load and concentrated load at the free end.

5.   	Compulsory:

A steel gun barrel is subjected to an internal pressure of 70Mpa. The internal   diameter of the barrel is 75mm and external diameter of 225mm. A steel band 25mm thick and internal diameter 0.075mm smaller than the external diameter of the gum barrel is shrunk on to the gun barrel, calculate

(a) the shrinkage pressure on the gun barrel

(b) maximum stress in the steel band, and

(c) minimum temperature to which the band must be heated to make assembly.

For steel , E= 200Gpa, ν = 0.3 and coefficient of thermal expansion = 10 x 10 - 6 / o C

6. 	a.	A square shaft rotating at 250rpm transmits torque to a crane which is designed to lift

maximum load of 150kNat a speed of 10m/min.  If the efficiency of a crane gearing is 65% estimate the size of the shaft for the maximum permissible shear stress of 35Mpa. Also calculate the angle of the twist of the shaft for a lenghth of 3m. Take G= 100Gpa

[P.T.O]

b.	An elliptical shaft of semi axes α = 0.05, b= 0.025m and G= 80Gpa is subjected to a twisting

moment of 1200π Nm . Determine the maximum shearing stress and angle of twist per unit length

(OR)

7.  	Derive an expression for torsion for a thin rectangular section using membrane analogy method

8. 	Describe the various theories of strength of plastic flow

(OR)

9.	a.	A flat plate is 2m long , 1m wide and 10mm thick . If uniform tension is applied only in X-

direction and the length increases to 2.5m without changing its width. Determine

i.	finite principal strains

ii.	find dimensions of the plate

iii.	Total effective strain by neglecting elastic strains

b.	The state of stress at a point is given by

500		500	  -400

T  ij   = 			500	      -300	   300		MN/ m2

-400	300	   -100

Calculate   a) Mean stress      b) Effective stress