Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	STABILITY OF STRUCTURES						              Time : 3 hours

Subject Code: 	CE332			         						                Maximum Marks: 100

Stability Functions Chart / Table is permitted

Assume any missing data suitably.

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

A simply supported rectangular plate of size (2a x a) is subjected to uniform axial compressive force of intensity Nx per unit length along the shorter edges. Determine the critical load at which the plate buckles.

2.	a.	Explain why stability problems are called as an eigen value problem.				(8)

b.	Determine the buckling load of an axial member with one end fixed and the other free.	(12)

(OR)

3.	Determine the critical load of a simply supported column subjected to axial load P. The moment of inertia varies from I0 at one support to 2I0 at the other support. Adopt Energy method.

4.	Obtain expression for the maximum deflection and maximum moment of a beam – column whose ends are simply supported and is loaded with uniformly distributed load of intensity ‘q’ per  unit length and an axial load of ‘Q’ at the ends.

(OR)

5.	a.	Obtain an expression for double modulus theory stating the assumptions.			(10)

b.	For a rectangular section of depth ‘h’ and width ‘b’ obtain the double modulus value.	(10)

6.	a.	Explain Flexural – Torsional buckling with examples.								(8)

b.	A cantilever bream of uniform cross-section and length ‘L’ is subjected to a lateral load ‘P’ at

the free end through the centroid of the section. Find its buckling load.				(12)

(OR)

7.	a.	Distinguish between flexural and torsional buckling.								(6)

b.	Determine the length of the column at which the torsional buckling load will be equal to the 	flexural buckling load of symmetrical I – section made with plates – Flagnes 100mm x 2mm 	and web 200mm x 2mm, so that the overall depth is 204mm. Take E = 200 GPa and                 	G = 80 GPa.																	(14)

8.	Obtain the buckling load for the symmetrical buckling of a cylindrical shell under the action of uniform axial compression.

(OR)

9.	Write short notes on the following:

a.	Imperfection sensitive and insensitive systems.									(7)

b.	Bifurcation and limit points.													(7)

c.	Torsional buckling of cylinders.												(6)