Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	ASEISMIC DESIGN OF STRUCTURES						 Time : 3 hours

Subject Code: 	CE334			         						                Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a.	State the orthogonal property of mode shapes.									(4)

b.	What do you mean by decoupling of equations in dynamics?						(6)

c.	Determine by Rayleigh’s method, the fundamental frequency of the system

shown in figure 1																(10)

Figure 1

2.	a.	A tower is subjected to a dynamic load as shown in figure 2. Evaluate numerically by

Duhemel’s integral and its response											(16)

Figure 2

b.	Why Rayleigh's method leads to higher values for the frequency?				(4)

(OR)

3.	a.	Discuss the response of a structure to blast loading							(4)

b.	A steel rigid frame  as shown in figure 3 supports a rotating machine. The machine exerts a

horizontal force of 50,000 sin11t. Assuming 4% critical damping, calculate the steady state amplitude of vibration. I for columns = 1500x10-7 m4, E = 21x1010 N/m2 		(16)

Figure 3

4.	a.	What are the major changes in the revised code of IS 1893 (Part I) : 2002? 			(8)

b.	Write briefly about seismic retrofitting and sketch the detailing requirement for the special confining reinforcement in footings												(12)

(OR)

5.	Write briefly about

a.	Irregular buildings and torsion													(8)

b.	Write down the expression for design horizontal seismic coefficient Ah as given in

IS : 1893 - 2002 and explain the terms.											(6)

c.	Active control system															(6)

6.	a.	Explain the principle of capacity design?										(8)

b.	A block of 10 storeyed flats in chennai has its lower most column 500x700 mm in size. In order to use the ground floor for car parking, the lower columns are free standing. Design the column as per IS 13920. use M25 grade concrete and Fe 415 steel. Sketch the reinforcement details.															(12)

(OR)

7.	a.	Write step by step design procedure for the design of reinforced concrete shear wall.  	(8)

b.	A beam AB of 5 m span is to be designed for moments MA = -60 kNm and +30 kNm,     MB = -76 kNm and +15 kNm. The characteristic dead and live loads are 5 kN/m and            8 kN/m respectively. The structure is situated in zone IV. Design the beam as per IS 13920. use M 25 grade of concrete and Fe 415 steel.										(12)

8.	Briefly discuss the following:

a.	Strengthening techniques for existing RC Buildings								(8)

b.	Discuss the performances of a base isolated structures and a structure incorporating viscous dissipating devices under seismic excitation.										(12)

(OR)

9.	Write notes on the following:

a.	Concept of base isolation systems												(8)

b.	Different types of passive control systems with neat sketches						(12)