Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	PRESTRESSED CONCRETE STRUCTURES		              Time : 3 hours

Subject Code: 	CE335			         						               Maximum Marks: 100

#### Assume suitable data wherever necessary

Reference to IS : 1343 – 1980 is permitted

#### Answer ALL questions (5 x 20 = 100 Marks)

1.	A post tensioned concrete beam of span 10m has a section 300 x 800mm. The beam is prestressed by a parabolic cable concentric at the supports and with a eccentricity of 250mm at the centre of the span. The cross sectional area of high tensile wires in the cable is 500mm2. The wires are stressed using a jack at the left end such that the initial prestressing force in the cable at the right end is 250 kN, using the following data, calculate the total loss of stress in the wires and the jacking force required at the left end. Coefficient of friction for curvature effect = 0.55.

Friction coefficient for wave effect = 0.003m

Anchorage slip at jacking end = 3mm

Relaxation of steel = 4%

Shrinkage of concrete = 0.0002

Es = 210 kN/mm2

Ec = 35 kN/mm2

(OR)

2.	Find the maximum allowable central concentrated load on a simply supported prestressed concrete beam given the following particulars.

Span = 12m

Section – unsymmetrical I section

Width of top flange = 400mm

Width of bottom flange = 300mm

Width of web = 150mm

Thickness of top flange = 100mm

Thickness of bottom flange = 200mm

Total depth of beam = 800mm

HTS wires are used – 20 numbers of 7mm diameter at 150 mm from the bottom edge.

Permissible compressive stress in concrete = 14 MPa

Permissible tensile stress in concrete = 1.0 MPa

Loss in prestress = 20%

Initial stress in steel = 1200 MPa

3.	A prestressed beam of rectangular cross section 300mm x 500mm is simply supported on a span of 10m. It is provided with a straight cable at an uniform eccentricity of 150mm below the beam axis throughout. There is also another cable which has an eccentricity of 60mm above the beam axis at mid span. The initial prestressing force in each cable is 600 kN. M 60 grade of concrete is used. Determine.

a.	the maximum short term deflection due to self weight and prestressing force.

b.	the maximum long term deflection under a working load of 120 kN at mid span. The loss of

prestress = 20 percent and creep coefficient = 1.60.

c.	check the above deflections as per codal requirements.

[P.T.O]

(OR)

4.	A prestressed concrete beam of section 200mm x 400mm is prestressed by a parabolic cable with an eccentricity of 100mm at the mid-span and zero at the supports. The prestressing steel has an area of 400mm2 and carries an effective prestress of 900 N/mm2. The beam carries a live load of 8 kN/m over a clear span of 12m. The characteristic strength of concrete is 50 MPa. Calculate the ultimate shear resistance of the section at the support and design the necessary shear reinforcement. Adopt IS: 1343-1980 provision.

5.	A post tensioned beam of depth 600mm and 300mm width has a span of 13m. It is provided with a parabolic cable having a stag of 125mm at mid span and zero eccentricity at ends. The initial prestressing force in the cable is 250 kN/m. The loss ratio is 0.85. The beam supports live load of 12 kN/m. Check the shear resistance of the beam at a distance of 1.5m from the supports. Design the necessary shear reinforcements at this section. M50 grade of concrete is used.

(OR)

6.	A composite T girder of span 5m is made up of a pre-tensioned rib, 100mm wide by 200mm deep, with an in situ cast slab, 400mm wide and 40mm thick. The rib is prestressed by a straight cable having an eccentricity of 33.33mm and carrying an initial force of 150 kN. The loss of prestress may be assumed to be 15%. Check the composite T beam for the limit state of deflection if it supports and imposed load of 3.2 kN/m for (a) unpropped construction, and (b) propped construction. Assume a modulus of elasticity of 35 kN/mm2 for both precast and in situ cast elements

7.	Compulsory:

a.	Write a note on methods of achieving continuity between two precast beams.		(10)

b.	Explain the following terms:													(10)

i.	Resultant moment		ii.	Transformation profile

8.	A cylindrical prestressed concrete water tank of internal diameter 30m is required to store water over a depth of 7.5. The permissible compressive stress in concrete at transfer is 13 N/mm2 and the minimum compressive stress under working pressure is 1 N/mm2. The loss ratio is 0.75. Wires of 5mm diameter with an initial stress of 1000 N/mm2 are available for circumferential winding and Freyssinet cables made of 12 wires of 8mm diameter stressed to 1200 N/mm2 are to be used for vertical prestressing. Design the tank walls assuming the base as fixed. The cube strength of concrete is 40 N/mm2.

(OR)

9.	Write a note on main advantages of prestressed concrete poles.