Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	ADVANCED DESIGN OF REINFORCED CONCRETE STRUCTURES													              						Time : 3 hours

Subject Code: 	CE364			         						                Maximum Marks: 100

Use of IS 456, IS1893, BS 110 (part I), ACI 318, SP 16 (S & T),

SP 24 (S & T) is permitted

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

Determine the reinforcement necessary for a rectangular beam section of width 300mm and depth 600mm which is subjected to Factored BM 100 kN-M, Factored torsional moment

50 kN-m together with a Factored shear force of 80 kN. Use M – 20 grade concrete and Fe – 415 grade steel.

2.	Design a bunker to store 500 tonnes of coal. The unit weight and angle of repose of coal may be taken as 8100 N/m3 and 30 respectively. The stored coal is surcharged at its angle of repose.

(OR)

3.	Design a short column of size 350mm x 450mm subjected to a Factored load of 1200 kN and biaxial factored moments Mux = 100 kN – m, and Muy = 80 kN – m, use M25 concrete and

Fe -415 steel.

4.	A rectangular slab 4m x 5m is simply supported at its ends. Design the slab to carry super-imposed service load of 5 kN/m2, if the slab is to be isotropically reinforced. Use M 20 concrete and Fe 415 steel.

(OR)

5.	a.	Explain Hillerborg’s simple strip method of analysis.								(10)

b.	Describe the design of flat slabs according to Equivalent frame method.				(10)

6.	a.	Discuss the conditions for Moment redistribution as per IS 456 and BS 8110 codes.	(10)

b.	Explain the methods for moment redistribution for a two-span continuous beam.		(10)

(OR)

7.	The tee beam ABC is continuous over two spans of 8m each and its carries factored u.d.l of

75 kN/m. Assuming fck = 25 N/mm2 and fy = 415 N/mm2, check whether we can reduce the maximum moment by 30% and redistribute to the spans. The tee beam has flange of width 1000mm, width of web 300mm, thickness of slab 150mm and overall depth of 820mm.

8.	Determine the thickness and reinforcements for a simply supported transfer girder of length 5.25m loaded from two columns at 1.75 m from each end with 3750 kN. The total depth of the beam is 4.2m and the width of supports is 520 mm. Assume grades of concrete M 40 and steel

Fe 415.

(OR)

9.	Design a Corbel for a 300mm square column to support a vertical ultimate load of 550 kN, with its line of action 200mm from the face of the column. Assume the grade of concrete M 20 and that of steel Fe 415.