Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	ADVANCED DESIGN OF STEEL STRUCTURES	              Time : 3 hours

Subject Code: 	CE365			         						                Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

A column of effective height 6.5m carries end moments of 25 kNm about the major axis and

10 kNm about the minor axis, in addition to the axial force of 350 kN, design the column. Take Cm = 1 for bending about major axis and 0.6 for bending about the minor axis.

2.	Design a Gantry girder for the following specifications

Span of the bridge = 10m

Weight of the bridge = 8T

Spacing of columns = 5m c/c

Wheel base distance = 3m c/c

Capacity of the crane = 6T

Weight of the crab = 2T

Edge Clearance = 1.1m

(OR)

3.	The span of knee braced roof trusses used over an industrial building 30m long is 16m. The spacing of roof trusses is 4m. The pitch of roof truss is 1 in 4. The galvanized corrugated iron sheets are used for roof covering. The basic wind pressure is 1.5 kN/m2 and there is no snowfall. The height of eaves above ground level is 8m. Propose a suitable type of roof truss. Determine the loads at the various points due to dead load, live load and the wind load. Determine the reactions also.

4.	Analyse the frame shown in Fig. 1 by portal method.

100 kN

150 kN

3.5m

3.5m

3.5m

7 m

5m

A

B

C

D

E

F

G

H

I

J

KG

L

(OR)

5.	Analyse the frame shown in Fig. 2 by cantilever method

[P.T.O]

120kN

180kN

4m

4m

4m

3m

4m

A

B

C

D

E

F

G

H

I

J

KG

L

Fig.2

6.	What are the different types of tower recommended by IS 802-1977 and also explain with sketches the different tower configurations and bracings

(OR)

7.	A 60m high microwave antenna lattice tower is to be built near Agra where the terrain at the site is nearly a level ground with terrain of category 2. The diameter of the hemispherical antenna disc, fixed at the top is 3m. The width of the tower at the top is 3.5m. Select a suitable configuration for the tower and determine maximum compressive force and tension in the tower legs and also the maximum shear at the base, for the following data:

Weight of antenna disc and fixtures	:	 9 kN

Weight of platform at top			:	 0.82 kN/m2

Weight of railing at top			:	 0.3 KN/m2

Weight of the ladder and the cage	:	0.65 kN/m

Weight of miscellaneous items	:	:	2.5kN

8.	a.	What are the different mechanics of failures? Explain 						(6)

b.	Explain how plastic hinges are formed.										(6)

c.	Determine the Collapse Load for a three span continuous beam subjected to uniformly

distributed load 															(8)

(OR)

9.	Determine the value of W at collapse for the portal frame shown in Fig.3.  All the members have the same plastic moment of resistance

2W

W

½L

½L

L/2

Fig. 3