Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	PRINCIPLES OF CHEMICAL ENGINEERING		              Time : 3 hours

Subject Code: 	CL202			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Convert  -40˚C to  ˚F

2.	State Law of conservation of energy.

3.	What is tie material?

4.	What is purge?

5.	Define Latent heat of vaporization.

6.	Define Specific heat.

7.	Give two examples for non Newtonian fluids.

8.	What is streak line?

9.	High flow rate and low pressure rise pumps are generally _____________.

10.	State Stoke’s Law.

PART – B  (5 x 3 = 15 MARKS)

11.	Aqueous solution of triethanolamine (TEA) ie.,N(CH2CH2OH)3, contains 50% TEA by mass. Find the molarity of the solution if the density of the solution is 1.05 kg/L.

12.	An aqueous solution of NaCl contains 30% salt by mole at 298 K. Find the composition of the solution by mass %? (Molar mass of NaCl = 58.4428)

13.	Define Bubble and Dew points.

14.	Determine the viscosity of a liquid having kinematic viscosity 6 stokes and specific

gravity 1.9 (333)

15.	Compare centrifugal pumps and reciprocating pumps.

PART – C  (5 x 15 = 75 MARKS)

16.	Water is flowing through a pipeline 6 cm in diameter. The local velocities at various positions are given below.

| U, cm/s | 2.00 | 1.94 | 1.78 | 1.50 | 1.11 | 0.61 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| R, cm | 0 | 0.5 | 1.0 | 1.5 | 2.0 | 2.5 | 3.0 |

Estimate the average velocity by (i) trapezoidal rule and (ii) Simpson’s one third rule.

(OR)

17.	a.	A solution of sodium chloride in water contains 25% NaCl (by mass) at 333 K. The density

of the solution is 1.127 kg/L. Calculate the molarity, normality and molality                         of the solution. 																(8)

b.	Calculate the specific volume of superheated steam at 100 bar and 623.15 K using the

vander Waals equation. 														(7)

[P.T.O]

18.	a.	A triple effect evaporator with forward feed is used to evaporate 13000 kg/hr of an aqueous

solution having an initial mass 9% to the final concentration of 43%. Calculate the concentration of the solution after each effect and also the quantity of solvent evaporated if it is known that 10% more water is evaporated in each effect than in the preceding one. (10)

b.	Define conversion and selectivity. 												    (5)

(OR)

19.	Chlorobenzene is nitrated using a mixture of nitric acid and sulphuric acid. Feed consists of    	100 kg chlorobenzene, 106.5 kg 65.5% nitric acid and 108 kg 93.6% sulphuric acid. After two 	hours of operation, the final mixture was analysed. It was found that the final product contained 	2% unreacted chlorobenzene. Also, the product distribution was found to be                                	66% p-nitrochlorobenzene and 34% 0-nitrochlorobenzene. Calculate (a) the analysis of feed,     	(b) 	the percentage conversion of chlorobenzene and (c) the composition of the product. (All % 	in wt)

20.	a.	Define Heat capacity, calorific value, latent heat, sensible heat and residual

heat capacity. 																(10)

b.	Write down the steps involved in energy balance with chemical reaction. 			(5)

(OR)

21.	1000 kg/hr of thermic fluid, to be used as a heat transfer medium, is being indirectly heated in a heater from 380 K to 550 K. Calculate the heat load on the heater, in KW. Also estimate the mean heat capacity of the thermic fluid over the temperature range of interest. The heat capacity equation for the thermic fluid is: Cp = 1.436 + 0.00218T, where Cp is in kJ/kg.K and T is in K.

22.	a.	A 25 cm diameter pipe carries oil of specific gravity 0.9 at a velocity of 3 m/s. At another

section the diameter is 20 cm. find the velocity at this section and also mass rate of flow of oil. 																			(10)

b.	List out the assumptions made in the derivation of Bernoulli’s Equation. 			(5)

(OR)

23	a.	Derive an expression for the discharge through Venturimeter. 						(10)

b.	Write down the Continuity equation and Bernoulli’s Equation. 					(5)

24.	a.	Discuss the applications of fluidization. 											(5)

b.	With neat diagram explain the working principle of centrifugal pumps. 				(10)

(OR)

25.	a.	Compare the advantages and disadvantages of centrifugal pumps with

reciprocating pumps. 															(7)

b.	Explain the characteristics curves of centrifugal pumps. 							(8)