Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	UNIT OPERATIONS								              Time : 3 hours

Subject Code: 	CL204			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is flow number?

2.	What is agitation?

3.	Give two examples for continuous filters.

4.	Diatomaceous earth is used in filtration as _____________.

5.	The Fourier Law Q = -K.A.dT/dX assumes _______________.

6.	Transfer of heat energy takes place in accordance with _______ Law of thermodynamics.

7.	Define Prandtl Number.

8.	Give example for natural convection heat transfer.

9.	What is pitch in heat exchangers?

10.	What is the use of baffles in shell and tube heat exchangers?

PART – B  (5 x 3 = 15 MARKS)

11.	List out of the purposes of agitation.

12.	How filters are classified based on filtration mechanism?

13.	What is Biot Number? Give its significance.

14.	Write down the different regimes in boiling.

15.	Write down the classification of heat exchangers.

PART – C  (5 x 15 = 75 MARKS)

16	a.	With neat diagram, explain the design principles of an agitated vessel.                 		(10)

b.	How to prevent swirling in agitated vessel?                                                             	(5)

(OR)

17.	Write notes on agitator selection and scale – up criteria.

18.	With neat diagram, explain the operating principle of plate and frame filter press.

(OR)

19.	Explain the Kynch theory of sedimentation.

20.	A composite wall consists of three layers of thicknesses 300 mm, 200 mm and 100 mm with thermal conductivities 1.5, 3.5 and K3 W/m.K respectively. The inside surface is exposed to gases at 1200˚C with convection heat transfer coefficient as 30 W/m2.K. The temperature of air on the other side of the wall is 30˚C with convective heat transfer coefficient 10 W/m2.K. If the temperature at the outer surface of the wall is 180˚C, calculate the temperature at the other surface of the wall, the thermal conductivity (K3) and the rate of heat transfer.

(OR)

21.	a.	Derive an expression for evaluating the temperature variation in a solid using lumped

capacity analysis.                                                                                                             (10)

[P.T.O]

b.	Derive an expression for the heat transfer rate through a composite cylindrical system. (5)

22.	Using dimensional analysis, prove that, for laminar flow forced convection

Nu = f(Re, Pr)

(OR)

23.	a.	Discuss the factors affecting Nucleate boiling. 									(7)

b.	 Define: (i) Emissive power; (ii) Radiosity; (iii) Transmissivity; (iv) Absorptivity. 	(8)

24.	a.	Draw 2-4 shell and tube heat exchanger and mention its parts.                              		(10)

b.	Explain the design procedure for double pipe heat exchanger.                                	(5)

(OR)

25.	In a double heat exchanger, hot water flows at a rate of 5000 kg/hr and gets cooled from 95˚C to 65˚C. At the same time 5000 kg/hr of cooling water enters the heat exchanger. The overall heat transfer coefficient is 2270 W/m2.K. Calculate the heat transfer area and the efficient assuming two streams are in (i) parallel flow and (ii) counter flow. (Take Cp for water is 4.2 kJ/kg.K, cooling water inlet temperature is 30  ̊C)