Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	CHEMICAL REACTION ENGINEERING			              Time : 3 hours

Subject Code: 	CL205			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	__________ states that for many reactions and particularly elementary reactions, the temperature dependency of reaction rate constant K, can be correlated by K = Kooe-E/RT

2.	From  the material balance equation

Input = output + disappearance by reaction + accumulation

If      input = output = 0  the reactor is

A. PFR         B. Batch        C.CSTR

3.	In _____________reactor, one of the reactants (say A) is charged initially and the other reactant (say B) is added continuously to the reactor over a certain time period under agitation. After addition is over, holding of reactor mass is done for certain time period and finally the product mixture is withdrawn from the reactor.

4.	Yield   is defined as _______

5.	RTD determined experimentally by using an inert chemical called a _________

6.	In _______ reactor the space time and mean residence time are same that the material spends in the reactor.

7.	A dimensionless number which tells whether the reaction under consideration is fast or slow and a film conversion factor,  assuming negligible gas phase resistance and considering unit gas-liquid interfacial surface is known as _______

8.	The roasting of sulphide ore to yield metal oxide is an example for

A Solid –fluid reaction    B.G/L reaction       C. Solid –solid reaction

9.	Effectiveness is defied as _________

10.	Thiele modulus is a dimensionless quantity used for ______

PART – B  (5 x 3 = 15 MARKS)

11.	After 8 minutes in a batch reactor, reactant  is 80% converted and after 18 minutes the conversion  is 90%. Find the rate expression to represent this reaction if C Ao = 1 mol/l .

12.	In an isothermal batch reactor 70% of  a reactant A  is converted  in 13 minutes. Find the space-time and space velocity needed to effect this conversion in MFR and PFR.

13.	Derive a relation between E,F and C curve for closed vessel.

14.	Derive a relation between time and radius and conversion if diffusion through ash layer controls in an unreacted core model for spherical particle of unchanging size.

15.	Explain the model proposed by Kunil and  Levenspiel for three zone mass transfer in fluidized bed reactor.

Bubble phase, Cloud phase and Emulsion phase

PART – C  (5 x 15 = 75 MARKS)

16.	Find the overall order of the irreversible reaction

2H2  + 2NO  			   	N2 + 2H2O

From the following constant volume data using equimolar amounts of hydrogen and nitric oxide

Total Pressure(mm Hg)     200      240     280    320    360

Half life(sec)                      265      186     115    104      67

(OR)

17.	a.	A  reaction 2HI	→   H2 + I2 is studied over a range of temperature  The results obtained are

as follows.																	(10)

| Temperature (K) | 633 | 666 | 697 | 715 | 781 |
| --- | --- | --- | --- | --- | --- |
| Rate constant | 1.7 x 10-5 | 1.07 x 10-4 | 5.01 x 10-4 | 1.05 x 10-3 | 1.51 x 10-3 |

- Find out the value of activation energy graphically using the given data

- Determine by what factor the rate increases when temperature raises from 300 to

310 K.																	(5)

b.	A common rule of temperature is that the rate of a reaction doubles for each 10 oC rise in temperature. What activation energy would this suggest at temperature of 25 oC   .		(5)

18.	a.	Derive the performance equation for PFR                            		           			(10)

b.	Differentiate holding time and space time for flow reactors.						(5)

(OR)

19. 	Consider the parallel decomposition of A of different orders.

R

A             S

T

with rR =1.0,  rS = 2CA, rT = CA2

Determine the maximum concentration of desired product (T) in

(i) PFR

(ii)MFR

Take CAo = 2

20. 	A reactor has few characteristics given by the non – normalized C-curve in table below, and by the shape of this curve we feel that the dispersion or tanks-in-series models should satisfactorily represent flow in the reactor.

| Time | Tracer concentration | Time | Tracer concentration |
| --- | --- | --- | --- |
| 1 | 9 | 6 | 86 |
| 2 | 57 | 8 | 77 |
| 3 | 81 | 10 | 67 |
| 4 | 90 | 15 | 47 |
| 5 | 90 | 20 | 32 |

- Find the conversion expected in this reactor, assuming that the dispersion model holds.

- Find the number of tanks in series which will represent the reactor and the conversion expected, assuming that the tanks in-series model holds.

(OR)

21.	A reactor with dividing baffles is to be used to carry the reaction

A  	  		R,  - r A  = 0.05CA ,

Pulse test results are:

| T , min | 0 | 10 | 20 | 30 | 40 | 50 | 60 | 70 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C | 35 | 38 | 40 | 40 | 39 | 37 | 36 | 35 |

Calculate conversion assuming (i) plug flow, (ii) assuming the tanks  - in series model, and (iii) assuming mixed flow.

22. 	The results of the kinetic runs on the reaction A    		R,   made in an experimental packed bed reactor using a fixed feed rate FAo  = 10 kmol/h are as follows.

| W, kg catalyst | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| X A | 0.12 | 0.20 | 0.27 | 0.33 | 0.37 | 0.41 | 0.44 |

a.	Find the reaction rate at 40% conversion

b.	For a feed of 400 kmol/h to large scale packed bed reactor, find the amount of catalyst needed for 40% conversion

c.	Find the amount of catalyst that would be needed in part (II) if the reactor employed a very large recycle of product stream

(OR)

23( i)   Explain the various steps involved in solid catalysed fluid phase reaction					(5)

ii) Define thiele modulus and derive the effectiveness factor for pore diffusion resistance combined with surface kinetics for single cylindrical pore. 								     (10)

24 (a) (i).Derive a general rate equation representing resistances in series in G/L reactions on a 	 		catalyst surface with a neat sketch.												    (10)

(ii)Explain the various ways of running G/L  reactions catalysed by solids.					(5)

(or)

25.  .Write short notes on

- Trickle bed reactor			(7)

- Slurry reactor				(8)