Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	PROCESS ENGINEERING CALCULATIONS		              Time : 3 hours

Subject Code: 	CL207			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The reactants present in excess is Termed as    _________________________.

2. 	Define Volume Percent?

3. 	Ideal gas law states that _______________.

4. 	Define Dalton’s Law?

5. 	What is Heat of mixing?

6. 	Unsteady state refers to__________________________.

7. 	Define recycle ratio.

8. 	Define stoichiometry.

9. 	Heat Capacity at constant pressure is defined as__________________.

10. 	Define Molority.

PART – B  (5 x 3 = 15 MARKS)

11.	Aqueous solution of triethanolamine (TEA), i.e., N (CH2CH2OH)3 , contains 50%  TEA by weight. Find the molarity of the equation of the solution if the density of the solution is 1.05 g/cm3.

12.	How many g moles of nitrogen will occupy 1000 m3 at 112x103 N/m2 and 400 K.

13.	The feed to a fractionating system is 30000 kg/hr of 50% benzene, 30% toluene and 20% xylene. The fractioning system consists of two towers No. I and No. II. The feed enters tower I. The overhead product from I is x kg/hr of 95% benzene, 3% toluene and 2% xylene. The bottom product from I is feed to II resulting in an overhead product of y kg/hr of 3% benzene, 95% toluene and 2% xylene while the bottom from II tower is z kg/hr of 1% benzene, 4% toluene and 95% xylene. Find x, y, and z.

14. 	Pure ethylene is heated from 30ºC to 250ºC at a constant pressure. Calculate the heat added per k mole.

15. 	A gaseous mixture analyzing CH4- 10%, C2H6-30% and rest H2 at 15ºC  And 1.5 atm is flowing through equipment at the rate of 2.5 m³/min. Find a)the average molecular weight of the gas mixture, b) Weight % and c.The mass flow rate

PART – C  (5 x 15 = 75 MARKS)

16.	The solution of NaCl in water contains 230 g of NaCl per litre at 20ºC.
The density of the solution at this temperature is 1.148 g/cc. Find the composition   in

a.	Weight %			b.	volume % of water		c.	mole %

d.	atomic %				e.	molality   and	  f.	g  NaCl/g water

(OR)

17.	The average molecular weight of a flue gas sample is calculated by two different engineers. One engineer used the correct molecular weight of N2 as 28, while the other used an incorrect value of 14. They got the average molecular weight as 30.08 and the incorrect one as 18.74. Calculate the % volume of N2 in the flue gases. If the remaining gases are CO2 and O2.  Calculate their composition also.

18.	a.	Calculate the weight of NaCl that should be placed in a 1 liter volumetric flask to prepare a Solution

of 1.8 molality. Density of this solution is 1.06 g/cc.

b.	A mixture of NH3 and air at 730 mm Hg and 30ºC contains 5.1 NH3 by volume. This gas is passed at a rate of 100 ft³ /min. through an absorption tower in which NH3 is removed. The gases leave the tower at 725 mm Hg and 20ºC having 0.05% NH3 by volume .Calculate:

i.	the rate of flow  of gases leaving  the tower and      	       (ii) Weight of NH3 absorbed.

[P.T.O]

(OR)

19.	A mixture of NH3 and air at 730 mm Hg and 30ºC contains 5.1 % NH3..The Gas is passed through an absorption tower at the rate of 100 m³/hr where NH3 is removed. The gases leave the tower at 725 mm Hg   and 20ºC having 0.05 %NH3. Calculate

a.	the rate of flow gas leaving the tower 	b.	weight of NH3 absorbed in kg/hr

20.	 Carbon tetrachloride is made as follows:

CS2 +   3Cl2                 →            CCL4 + S2CL2

212.76                 		   153.84

The product gases are found to contain CCL4 33.3%; S2CL2 33.3 %; CS2 1.4% and CL2 32%. Calculate the following:

a) 	The percentage of the excess reactants used.		b)   the percentage conversion

c)   kg of CCL4 produced per 100 kg CL2 converted.

(OR)

21.	In order to obtain barium in a form that may be put into solution, the natural sulphate (barites) containing only pure barium sulphate and infusible matter is fused with an excess of pure anhydrous soda ash. Upon analysis of the fusion mass it is found to contain 11.3% barium sulphate, 27.7% sodium sulphate and 20.35% sodium carbonate. The remainder is barium carbonate and infusible matter. Calculate the following:

(a)  	% conversion of barium sulphate to the carbonate.  (b) Composition of the original barites

(c) 	% excess of the sodium carbonate used in excess of the theoretically needed amount for all the

barium sulphate.

22.	a.	Find the heat of formation of ZnSO4 from its elements and from these					(7)

Data:                                                                  Kcal/mole

(i)	 ZnS   →   Zn + S                                       	          44

(ii) 	2ZnS + 3O2   → 2 ZnO + 2 SO2                     	 -221.88

(iii) 	2SO2 + O2 → 2SO3                                      		- 46.88

(iv) 	ZnSO4 → ZnO + SO3                                       	55.1

b.	Using the following data of heats of combustion in cal. /g mole, calculate the following: 	(8)

(i )	Heats of combustion of benzene to water           	(ii) Heat of vaporization of benzene

- cal. /g mole

(i)  C6H6 (l) to CO2 (g) and H2O (l)                                  = 7,80,980

(ii) C6H6 (g) to CO2 (g) and H2O (g)                                 = 7,59,520

(iii) H2 (g) to H2O (l)                                                          =    68,317

(iv) H2 (g) to H2O (g)                                                          =    59,798

(v) Graphite to CO2 (g)                                                      =     94,052

(OR)

23.	Find the theoretical flame temperature of a gas containing 30% CO and 70% N2 when burnt with 100% excess air. The reactants enter at 298 K.

∆Hf CO2 = -3, 93,7 00 kJ/k mole; ∆Hf CO = -1,10,600 kJ/k mole

Mean molar specific heat, kJ/k mole K

Temp, K                              CO2                                              O2                                           N2

800                                 	45.4                                               31.6                                         30.3

1000      	                   	47.6  		                      	       32.3                                         30.6

1200                              	49.4                                               33.0                                         31.2

1400                              	50.8                                               33.6                                         31.8

1600                               	52.0                                               34.0                                         32.3

1800                               	53.2                                               34.4                                        32.7

24.	Explain the various theories of reaction rates with suitable example.

(OR)

25.	Write short notes on: 															(7+8)

a.	Temperature dependence of reaction rates	b.	Enzymatic reaction