Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	UNIT OPERATIONS IN FOOD PROCESSING - I	              Time : 3 hours

Subject Code: 	CL208			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Rich cows milk (4536 kg/hr) at 4.4C is being heated in a heat exchanger to 54.4C by hot water. How much heat is needed? Specific heat: 12 kJ/kgK.

2.	Indicate few unit operations are necessary to extend the shelf life of product.

3.	When distillation is not recommended?

4.	Explain Fick’s lw of diffusion.

5.	What is Duhring rule.

6.	For the following mode of operation, give an advantage offered by it use.

Multiple – effect evaporator using forward feed.

7.	Selectivity of solvent used in extraction should be ________.

8.	Which extractor is not used for the emulsifying liquids?

9.	Give examples of supercritical fluid.

10.	Which solvent is currently employed in decaffeination of coffee?

PART – B  (5 x 3 = 15 MARKS)

11.	Show the schematic diagram of different component velocity in fluid mixing.

12.	What is the principle of steam distillation?

13.	Write the significance of D and Z value.

14.	What are the qualities to be considered while choosing a solvent for extraction?

15.	Write the important properties of supercritical fluid.

PART – C  (5 x 15 = 75 MARKS)

16.	A new formulation of meat product is obtained by using raw materials of lean beef, pork fat and water. Develop a mathematical model for the formulation to produce 100 kg of the new meat product. The data on raw material and requirement of new product are as under.

Lean beef : 14% fat, 67% water, 19% protein.

Pork : 89% fat , 8% water, 3% protein.

The requirement for new product of protein and water is 20% fat, 15% protein and 65% water.

(OR)

17.	Write notes on the operation and functioning of important unit operations in the food industry.

18.	a.	Illustrate the mechanism of heat and mass transfer in deep potato frying.

b.	Explain flash distillation. How it differs from simple distillation?

(OR)

19.	A liquid mixture containing 40 mole% n-heptane and 60 mole % n – octane is subjected to differential distillation at atmospheric pressure, with 60 mole% of the liquid distilled. Compute the composition of the composited distillate and the residue.

Relative volatility for given system is 2.16.

[P.T.O]

20.	With the help of neat sketches, discuss the classification of evaporators.

(OR)

21.	a.	A triple effect evaporator is concentrating a liquid that has no appreciable elevation in

boiling point. The temperature of the steam to the first effect is 108C and the boiling point of the solution in the last effect is 52C. The overall heat transfer coefficients, in W/m2 C are 2500, 2000 and 1500 in the first, second and third effect. At what temperature will the liquid boil in the first and second effect?

b.	Differentiate Capacity and Economy.

22.	Explain the working principle of the following extractors with a neat sketch and state its applications.

a.	Pulse Column extractor			b.	Rotor agitated column extractor

(OR)

23.	a.	Explain the Primary and Secondary nucleation in crystallization.

b.	Discuss the mechanism of leaching and the factors which influence the rate of leaching.

24.	Discuss the principles of supercritical fluid extraction. Explain its potential applications in food industry.

(OR)

25.	a.	Explain the advantages and limitations of supercritical fluid extraction.

b.	Show the phase diagram and explain the supercritical region of solvent.