Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	HEAT TRANSFER AND FLUID FLOW				               Time : 3 hours

Subject Code: 	CL209			         						                Maximum Marks: 100

Use of Heat Transfer Data Book Permitted

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Reynolds number is given by …..............

2.	The friction factor is defined as …............

3.	Distinguish between notch and weir.

4.	What is an orifice?

5.	Give two examples for conduction with internal heat generation.

6.	At a given temperature a gray surface emits less radiation than a ….......surface.

7.	Give the expression for the drag coefficient.

8.	Heat transfer in laminar flow is ….................. than in turbulent flow.

9.	The dimensions of thermal conductivity in MLT system is ….........

10.	Peclet number is used in …...................

PART – B  (5 x 3 = 15 MARKS)

11.	Write the Bernoulli's equation for flow with friction.

12.	What is a hot wire anemometer? Explain its function.

13.	Write the expressions for thermal resistance of a cylinder and a sphere.

14.	Draw the velocity boundary layers for internal and external flows and mark the regions.

15.	State the conditions for similarity between model and prototype.

PART – C  (5 x 15 = 75 MARKS)

16. 	Water flows at the rate of 200 l/s upwards through a tapered vertical pipe. The diameter at the bottom is 240 mm and that at the top is 200mm and its length is 5 m. The pressure at the bottom is 8 bar and that at the top is 7.3 bar. Determine the head loss through the pipe. Express it as a function of exit velocity head.

(OR)

17. 	Derive an expression for the loss of head due to friction in pipe flow.

18. 	A pipe 10 cm in diameter has a nozzle attached to it at the discharge end.  The diameter of the nozzle is 5 cm. The rate of discharge of water through the nozzle is 2 l/s and the pressure at the base of the nozzle is 0.59 N/cm2. Calculate the coefficient of discharge. Assume that the base of the nozzle and the outlet are at the same elevation.

(OR)

19.	Write detailed notes on rotameters and hot wire anemometer.

20.	Starting from principles derive an expression for the temperature distribution and rate of heat transfer through a solid cylinder subjected to steady conduction heat transfer with constant rate of internal heat generation.

[P.T.O]

(OR)

21.	A steel ball [c=0.46 kJ/kg-K, k=35 W/m-K] 5 cm in diameter and initially at a uniform temperature of 450o C is suddenly placed in a convection  	 environment in which the temperature is maintained at 100o C. The convection heat transfer coefficient is 10 W/m2-K. Calculate the time required for the ball to attain a temperature of 150o C. Justify the method used for solution.

22.	Air at 20 deg. C and 1 atm flows over a flat plate at 40 m/s. The plate is 80 cm long and is maintained at 60 deg. C. Assuming unit width, calculate the heat transfer rate. Use any two different correlations for Nu and compare the answers.

(OR)

23.	a.	Discuss the analogy between heat transfer and fluid friction

b.	Write detailed account on turbulent heat transfer and Prandtl's hypothesis.

24.	Explain in detail the method of obtaining dimensionless parameters using Buckingham theorem.

(OR)

25.	Write short notes on: a] dimensionless numbers in heat transfer and b] model testing.