Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	UNIT OPERATIONS IN FOOD PROCESSING - II	              Time : 3 hours

Subject Code: 	CL210			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Write the Langmuir isotherm and its significance.

2. 	Name few unit operations where diffusion plays an important role?

3. 	Give examples of filter aids.

4. 	Write an equation for estimating the pressure drop through cake during a batch filtration process.

5. 	Show the schematic diagram of tubular bowl centrifuge.

6. 	Give examples of flocculants and coagulants.

7. 	Name few polymers used for reverse osmosis membranes.

8. 	Indicate the applications of ultra filtration

9. 	Classify the extruders.

10.	Write the applications of roasters in food industries.

PART – B  (5 x 3 = 15 MARKS)

11. 	Explain the Fick’s law of diffusion.

12. 	List factors that affecting the specific cake resistance in filtration and how to increase the filtration efficiency.

13. 	Write the advantages of centrifugation over the sedimentation.

14. 	Explain rejection coefficient and its significance.

15. 	How an extruder works?

PART – C  (5 x 15 = 75 MARKS)

16. 	a. 	Explain the applications of diffusion and migration in food packaging.

b. 	Write the theoretical considerations of diffusion in porous foods.

(OR)

17. 	We have measured the adsorption of the enzyme papain from water at pH 9.5 onto a adsorbent. We find the following results:

| y(mg/liter) | 0.01 | 0.02 | 0.03 | 0.06 | 0.10 |
| --- | --- | --- | --- | --- | --- |
| q(mg/cm3) | 5.3 | 7.1 | 8.3 | 11.1 | 13.7 |

What adsorption isotherm fits these data?

18. 	a. 	Explain the working of a plate and frame filter press with the help of a neat sketch.

b. 	A rotary filter operating at 0.03 Hertz , filters 0.075 m3/sec .Operating under the same vacuum  and neglecting  the resistance  of  the filter cloth ,at what speed must the  filter be operated  to give a  filtration  rate of  0.016 m3/sec

(OR)

19. 	Starting from Kynch theory, explain the method for determining the area of a continuous thickener and list the different types of thickeners.

[P.T.O]

20. 	Broadly describe the principles of operation of different types of centrifuges.

(OR)

21. 	Spirulina cells are being cultivated in an open pond. We plan to harvest this biomass by passing the dilute stream of cells through an available disc bowl centrifuge. The settling velocity vg for these cells has been measured as 1.07 * 10-4 cm/sec. The centrifuge has 80 discs with an angle of  40, an outer radius of 15.7 cm, and an inner radius of  6 cm. we plan to operate the centrifuge at 6000r/min. Estimate the volumetric  capacity Q for this centrifuge .

22. 	Explain the different types of membrane configurations in ultrafilteration that are widely used for large-scale purification.

(OR)

23. 	A reverse osmosis membrane to be used at 25 C for a NaCl feed solution containing                     2.5 kg NaCl/L has a water permeability constant AW = 4.8 *10-4 Kg/s.m2.atm   and solute permeability constant As = 4.42 *10-7 m/s. calculate the water flux and solute flux through the membrane using  p = 27.5 atm  and  = 2atm . Also find solute rejection and c2 of the product solution.

24.  	a. 	What is meant by extrusion cooking and its applications?

b. 	Describe the different kinds of extruders.

(OR)

25. 	With schematic diagram, explain the different kinds of Roasters and its applications.