Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	FUNDAMENTALS OF THERMAL FLUID SCIENCES            Time : 3 hours

Subject Code: 	CL211			         						                Maximum Marks: 100

Any missing data may be suitably assumed.

Use of Heat and Mass Transfer Data book permitted.

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The unit of surface tension is __________.

2.	__________ equation deals with the law of conservation of mass.

3.	Define latent heat.

4.	Give the equation of state for an ideal gas.

5.	Give the equation for Fourier’s law.

6.	Free convection is caused by __________ forces.

7.	The heat flux in nucleate pool boiling is higher for horizontal plane Say True/False.

8.	In laminar film condensation, the convection coefficient varies as ______________ with respect to plate height.

9.	NTU is defined as ________ .

10.	The three main flow arrangements in heat exchangers are _____________.

PART – B  (5 x 3 = 15 MARKS)

11.	Define Newton’s law of viscosity.

12.	State the Zeroth law of thermodynamics.

13.	Mention the factors influencing the amount of radiation.

14.	Where does flow boiling occur?

15.	What are fouling resistances?

PART – C  (5 x 15 = 75 MARKS)

16.	Define any six properties of fluids. Classify and explain the types of fluids.

(OR)

17.	A plate, 0.025 mm distant from a fixed plate, moves at 60 cm/s and requires a force of 2 N per unit area i.e; 2 N/m2 to maintain this speed. Determine the fluid viscosity between the plates.

18.	Explain how a cyclic heat engine exchanges heat with a source and a sink. Also explain the second laws of thermodynamics in detail.

(OR)

19.	Determine the quantity of steam to be supplied per hour to a water heater. The heater operates under steady flow conditions receiving 4.2kg/s of water at 75oC temperature, enthalpy 314 kJ/kg. The water is heated by mixing with steam which is supplied to the heater at temperature 100.2oC and enthalpy 2676kJ/kg. The mixture leaves the heater as liquid water at temperature 100oC and enthalpy 419 kJ/kg.

[P.T.O]

20.	A circular of pipe OD 20cm is enclosed centrally in a square section insulation of 36cm side. The thermal conductivity of the material is 8.5 W/mK. The inside surface is at 200oC. The outside is exposed to convection at 30oC with h=35W/m2K. Determine the heat flow per a length of 5m.

(OR)

21.	Determine the heat flow and overall heat transfer coefficient for the following problem: A composite wall is made of 12 mm and 18 mm layers of materials of thermal conductivity 12.5 and 22.5 W/mK. The contact resistance between surfaces is 5 X 10-4 m2 OC/W. The hot side is exposed to fluid at 400oC with h = 75W/m2K and the cold side is exposed to fluid at 60oC with h = 400W/m2K.

22.	Describe the nature of boiling process and the regimes of boiling.

(OR)

23.	Water at atmospheric pressure (saturation temperature = 100oC) is boiling on a brass surface heated from below. If the surface is at 108oC, determine the heat flux and compare the same with critical heat flux.

24.	Explain the thermal resistances encountered in heat transfer from one fluid to another. Give the necessary equations.

(OR)

25.	Determine the area required for a shell and tube heat exchanger with two tube passes to cool oil at a rate of 10kg/s from 60oC to 30oC flowing in the shell using water at 20oC passing through the tubes and heated up to 26oC. The specific heat of the oil is 2200 J/kgK. The value of heat of overall heat transfer coefficient is 300 W/m2K.