Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	UNIT OPERATIONS OF CHEMICAL ENGINEERING           Time : 3 hours

Subject Code: 	CL212			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	State the law of conservation of mass.

2.	Give the formula for Simpson’s 38rule .

3.	Give the equation for the overall material balance.

4.	Define absolute mass humidity.

5.	Identify the following dimensionless groups

(a)Pn3Da5ρ(b)nDa2ρμ

6.	State the purposes of agitation.

7.	Define terminal settling velocity.

8.	State the different types of drag.

9.	What are filter aids? Give examples.

10.	What is the use of doctor’s blade?

PART – B  (5 x 3 = 15 MARKS)

11.	Write the normal equations to fit a set of data to a parabola by the method of least squares.

12.	A mixture of carbon disulfide vapor and air contains 23.32 % CS2 by volume. Calculate the relative percent saturation and percent saturation of the mixture at 20°C and 740 mm Hg pressure. Vapor pressure of CS2 at 20°C = 300 mm Hg

13.	Derive the expressions for specific cake resistance and filter medium resistance for constant pressure filtration.

14.	Sketch a turbine and give the standard turbine dimensions.

15.	Explain the types of fluidization. State the advantages and limitations of fluidization.

PART – C  (5 x 15 = 75 MARKS)

16.	The relation between the friction factor, f, and Reynolds number Re, is of the form

f=aRem

| Re | 4530 | 5010 | 5780 | 9600 | 12600 | 15600 |
| --- | --- | --- | --- | --- | --- | --- |
| f | 0.0097 | 0.0095 | 0.0092 | 0.0081 | 0.0075 | 0.0071 |

Determine the best values of a and m.										(7.5 + 7.5)

(OR)

[P.T.O]

17.	The velocity of a particle at a distance S from a point on its path is given by the table below:

| S, m | 0 | 10 | 20 | 30 | 40 | 50 | 60 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| v, m/s | 47 | 58 | 64 | 65 | 61 | 52 | 38 |

Estimate the time taken to travel 60 m by using Simpson’s 1/3rd rule. Compare your answers with Simpson’s 3/8th rule and trapezoidal rule.								(5+5+5)

18.	For 1000 lit/s of a gaseous mixture of the following composition:

CH4 – 10 % ; C2H6 – 30 %; H2 – 60 % (all by volume) at 30°C and 2000 mm Hg (Gauge), calculate:

a.	mol fraction of each component											(2)

b.	weight fraction of each component											(3)

c.	the concentration of each component, gmmol/cc								(2)

d.	the partial pressure of each component										(2)

e.	the molar density of the mixture											(2)

f.	the mass flow rate of the mixture											(2)

g.	the average molecular weight of the gas.										(2)

(OR)

19.	Pure CO2 may be prepared by treating limestone with aqueous H2SO4. The limestone contains CaCO3 and MgCO3 and the remaining is insoluble matter. The acid is 12% H2SO4. The residue from the process had the following composition:

CaSO4 = 8.56%

MgSO4 = 5.23%

H2SO4 =1.53%

CO2 =0.12%

H2O = 84.51% and Inerts = 0.53%

If during the process the mass was warmed and CO2 and water vapor were removed, calculate:

(a)	composition of limestone used												(5)

(b)	% excess acid used														(5)

(c)	calculate the weight and analysis of the material distilled from the reaction mass per 1000 kg

of limestone treated.														(5)

20.	On the assumption that the power required for mixing in a stirred tank is a function of the following variables:

P = f (μ, ρ, N, g, D, DT, W, H)

Obtain the dimensionless groups which are important in calculating power requirements for geometrically similar arrangements.

(OR)

21.	Explain the following 														(7.5 + 7.5)

a.	Scaleup of agitators			b.	Flow patterns in agitated vessels

22.	a.	Derive the Kozeny- Carman equation. State your assumptions clearly. 			(8)

b.	A bed of ion-exchange beads 8 ft deep is to be backwashed with water to remove dirt. The particles have a density 1.24 g/cc and an average size of 1.1 mm. What is the minimum fluidization velocity using water at 20ºC? The beads are assumed to be spherical and minimum porosity is taken as 0.4. (Use m as 3.9). 							(7)

(OR)

23.	Derive an expression for terminal settling velocity of a spherical particle. Deduce the relations in laminar flow and turbulent flow. 												(8+7)

24.	a.	Data for the filtration of CaCO3 slurry in water at 25ºC are reported as follows: at a constant

pressure of 46.2 kN/m2. The area of the plate and frame filter press was 0.0439 m2. The slurry concentration was 23.47 kg/m3 filtrate. Calculate the specific cake resistance and the filter medium resistance.													(3.5 + 3.5 )

| V X 103, m3 | 0.5 | 1 | 1.5 | 2 | 2.5 | 3.0 |
| --- | --- | --- | --- | --- | --- | --- |
| Time, s | 17.3 | 41.3 | 72 | 108.3 | 152.0 | 201.7 |

b.	Explain the principle of working and applications of a continuous rotary vacuum drum filter

with a neat sketch.														(8)

(OR)

25.	a.	Explain differential settling. Obtain an expression for the ratio of the sizes of the particles

separated by differential settling both in laminar flow and turbulent flow.		(10)

b.	Explain in detail about gravity settling tanks.									(5)