Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	FUNDAMENTALS OF THERMAL FLUID SCIENCES FOR

FOOD ENGINEERS								               Time : 3 hours

Subject Code: 	CL215			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Define viscosity.

2. 	What is Newtonian fluid?

3. 	---------- properties do not depend on the mass of the system.

4. 	----------- cannot flow of itself from a body at a lower temperature to a body at a higher

temperature.

5. 	Write the equation of Fourier’s law of heat conduction.

6. 	A steady process is one, which is not dependent on ----------.

7. 	What is forced convection heat transfer?

8. 	What is critical Reynolds number for flow over a flat plate?

9.  	What are the two modes of condensation?

10. 	Boiling in a liquid at saturation temperature is known as ---------- boiling.

PART – B  (5 x 3 = 15 MARKS)

11. 	What are the various types of fluids? Differentiate between ideal and real fluids.

12. 	Explain what you understand by thermodynamic equilibrium.

13. 	Explain briefly the lumped capacity analysis in transient heat conduction.

14. 	Explain what you mean by Absorptivity, Reflectivity and Transmissivity.

15. 	What are the five significant dimensionless numbers in boiling and condensation?

PART – C  (5 x 15 = 75 MARKS)

16. 	State Bernoulli’s theorem for steady flow of an incompressible fluid. Derive an expression for Bernoulli’s equation and state the assumptions made for such derivation.							(OR)

17.	 a.	Explain the principle of venturimeter with a neat sketch.							(10)

b. 	A venturimeter with a 15 cm diameter at inlet and 10 cm diameter at throat is laid with its

axis horizontal and is used for measuring the flow of oil of specific gravity 0.9. The oil-mercury differential manometer shows a gauge difference of 20 cm. Assume co-efficient of the meter as 0.98. Calculate the discharge in litres per minute.						(5)

18.	a. 	Define thermodynamic system. Differentiate between open system, closed system   and an

isolated system.																(8)

b. 	Explain the first law of thermodynamics referred to closed systems undergoing a cyclic

change.																		(7)

(OR)

19.	a. 	What is steady flow process? Derive the SFEE for a single stream entering and    single

stream leaving a control volume and explain the terms in it.						(10)

[P.T.O]

b. 	Steam at a 6.87 bar, 205o C, enters in an insulated nozzle with a velocity of 50 m/s and

enthalpy of 2850 kJ/Kg. It leaves at a pressure of 1.37 bar and a velocity of 500 m/s. Determine the final enthalpy of steam.											(5)

20.	a. 	Describe the three modes by which heat can be transferred from one place to another. (10)	b. 	Explain briefly the overall heat transfer co-efficient.								(5)

(OR)

21.	a. 	A mild steel tank of wall thickness 12 mm contains water at 100o C. Calculate the rate of

heat loss per m2 of tank surface area when the atmosphere temperature is 20o C. The thermal conductivity of mild steel is 50 W/m K, and the heat transfer co-efficients for the inside and outside the tank are 2850 and 10 W/m2K, respectively. Calculate also the temperature of the outside surface of the tank.													(10)

b. 	Explain the significance of critical radius of insulation.							(5)

22 	a. 	State the five methods, which are available for evaluation of convection heat     transfer

co-efficient.																	(5)

b.	 A rectangular plate is 120 cm long in the direction of flow and 200 cm wide. The plate is

maintained at 80oC when placed in nitrogen that has a velocity of 2.5 m/sec and a temperature of 0oC. Determine (a) the average heat transfer co-efficient and (b) the total heat transfer from the plate. The properties of nitrogen at 40o C are  = 1.142 kg/m3; cp = 1.04 kJ/kg K, v = 15.63 x 10 – 6 m2/s and k = 0.0262 W/m K.							(10)

(OR)

23. 	a. 	Write short notes on (i) Black body and (ii) Emissive power.						(5)

b. 	A liquid metal flows at the rate of 4 kg/s through a constant heat flux 6 cm inner diameter tube in a nuclear reactor. The fluid at 200oC is heated with the tube wall 40oC above the fluid temperature. Determine the length of the tube required for 25oC rise in bulk fluid temperature using the following properties:  = 7.7 x 103 kg/m3,

cp = 130 J/kg K, v = 8 x 10 – 6 m2/s, Pr = 0.011 and k = 12 W/m K.					(10)

24. 	Describe the various regimes of pool boiling with a neat sketch.

(OR)

25.	a.	State the assumptions made in deriving Nusselt’s equation for film condensation. 	(5)

b. 	Explain the flow regimes in two-phase flow through a tube.						(10)