Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	PRINCIPLES OF CHEMICAL ENGINEERING		              Time : 3 hours

Subject Code: 	CL223			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Ratio of gram moles of solute to mass of solvent in kg is defined as __________

2.	For NTP conditions at 273.15K and 101.325kPa volume occupied by 1kmol of gas is

A. 22.4 m3/sec.  B. 20.4  m3/sec       C.  2.24 m3/sec

3.	A stream bled off to remove an accumulation of certain inerts or unwanted materials that might otherwise build up in recycle stream is called_______________

4.	In the reaction   A + B → C

C +D → D

Ratio of Moles of A reacted to produce C to moles of A totally reacted is known as

A. Yield     	B. Selectivity    	C. Conversion  	D. Excess reactance

5.	Stoke (St) is the representation of __________in cgs system.

6.	Pitot is an instrument used to measure ___________ Point velocity

7.	When fluid flows through a pipe line maximum velocity occurs at the walls of the pipe line.

(True /False )

8.	The wall thickness of pipe is indicated by _________number which increases with thickness.

9.	To avoid cavitation the pressure at the pump inlet must exceed the vapour pressure by a certain value______________

10.	Velocity head of the liquid from the impeller is converted into pressure head  in _____________pump

A. Centrifugal  	B. Rotary  	C. Plunger  	D. Diaphragm.

PART – B  (5 x 3 = 15 MARKS)

11.	State the following laws

(a)  	Daltans law

(b) 	Amagat s law

(c) 	Raoults law

12. 	Write the general and overall material balance with block diagram for a

(a) 	Distillation unit.

(b) 	Extraction unit

13. 	Discuss in detail about the rhelogical behavior of non-newtonian fluids.

14. 	Differentiate venture and orifice meter.

15. 	Explain the working principle of Air lift pump.

[P.T.O]

PART – C  (5 x 15 = 75 MARKS)

16.	Make the following conversions

(a) 294 g/l H2SO4 to normality

(b) 4.8mg/ml CaCI2 to normality

(c) 5 N H3PO4  to g/l

(d) 54.75 g/l HCl to molarity

(e) 3 M K2 SO4  to gm /lit

Data: Molecular weight of   H2SO4 =49, CaCl2 =55.5, H3PO4=49, K2 SO4 =174

(OR)

17. 	A gas mixture has the following composition by volume:

SO2= 8.5%, O2=10% and N2=81.5%

Find 	(a) Density of the gas mixture at a temperature at 473 K and 202.65kPa

(b) Composition by weight

Data: Molecular weight on SO2=64, O2=32, N2=28

18. 	In the Deacon process for the manufacture of chlorine a dry mixture of hydrochloric

acid gas and air is passed over a heated catalyst which promotes oxidation of acid.

Air is used 30% in excess of that theoretically required. kg of the acid. (Atomic

weight of chlorine = 35.5,air contains 23.2% oxygen by weight )

Reaction : 4 HCL + O2  → 2Cl2 + 2H2O.

Calculate the following

(a) 	The weight of air supplied per kg of acid.

(b) 	The composition (weight %) of gases entering.

(c) 	The composition (weight %) gases leaving, assuming that 60% of acid is oxidized in

the process.

(OR)

19. 	The producer gas made from the coke has the following composition by volume %                  CO-28, CO2 – 3.5, O2 – 0.5, N2 -68. The gas is burnt with such a quantity of air that  the oxygen from air is 20% in excess of the net oxygen required for complete  combustion. If the combustion is 98% complete calculate the weight of the gaseous product formed per 100kg of gas burnt.

20. 	a.	Prove that pressure at any point is independent of direction .                             	(7)

b.	Derive the mathematical statement for the condition of hydrostatic equilibrium. 	(8)

(OR)

21. 	a.	Show that in a laminar flow the velocity distribution with respect to radius is a

parabola with the apex  at the centerline of the pipe.                                       		(10)

b.	State Hagen Poiseuille equation and its application.                                         	(5)

22. 	a.	Discuss in detail about the mechanism and application of  fluidization .           	(7)

b.	Derive minimum fluidization velocity.                                                                	(8)

(OR)

23. 	a.	Explain the working principle of area flow meter and its calibration.               	(10)

b.	Differentiate orifice and venturi meter.                                                              	(5)

24. 	With a neat sketch explain the construction, performance and working principle of centrifugal 	pump.

(OR)

25. 	With a neat sketch explain the construction and working principle of positive displacement reciprocating pump.