Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	ADVANCED CHEMICAL ENGINEERING			              Time : 3 hours

Subject Code: 	CL303			         						                Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a. 	Explain in detail the electro dialysis process for removing Citric acid from grape

fruit juice.        																(10)

b.	What is sweep diffusion, and explain the process with suitable example. 			(10)

2.	a.	Carbon tetrachloride is made as follows:											(10)

CS2 +   3Cl2           →          CCL4 + S2CL2

  - 153.84

The product gases are found to contain CCL4 33.3%; S2CL2 33.3 %; CS2 1.4% and CL2 32%. Calculate the following: (i) The percentage of the excess reactants used (ii)   the   percentage conversion (iii)  kg of CCL4 produced per 100 kg CL2 converted.

b.	A contact sulphuric acid plant product 98% acid. A gas containing 8% SO3 (rest inert) enters a SO3 absorption tower at the rate of 28 k mole/hr. 98.5% of SO3 is absorbed in this tower by 97.3% acid introduced at the top and 95.9% acid is used as the make up acid. Compute tons /day of (i) make up acid required (ii) acid fed at the top of the tower and      (iii) Acid produced.															(10)

(OR)

3. 	a.	Pure ethylene is heated from 30ºC to 250ºC at a constant pressure. Calculate the heat added

per k mole Cp = 2.83 + 28.601 x 10 - 3T – 87.26 x 10 -7 T2    where Cp is in kcal/ k mole K and T in K. 																	(5)

b.	Find the heat of formation of ZnSO4 from its elements and from these

Data:                                                                                                      	 			(5)

Kcal/mole

(i) 	ZnS   →   Zn + S                                                 44

(ii) 	2ZnS + 3O2   → 2 ZnO + 2 SO2                      -221.88

(iii) 	2SO2 + O2 → 2SO3                                      - 46.88

(iv) ZnSO4 → ZnO + SO3                                       55.1

c.	The solution of NaCl in water contains 230 g of NaCl per litre at 20ºC. The density of the solution at this temperature is 1.148 g/cc. Find the composition in (i) atomic %  (ii) volume % of water (iii) mole % (iv) Atomic % (v) Molality   and  (vi)  g  NaCl/g water. 		(10)

4.	a. 	Derive Darcy equation for head loss due to friction.			 					(10)

b. 	A horizontal pipe, 10 cm in diameter, is joined by suddenly enlargement to a 15cm diameter pipe. Water is flowing through it at the rate of 2m3/min. Find the loss of head due to abrupt expansion and pressure difference in the two pipes. If the change of section is gradual without any loss, what would be the change in pressure?							(10)

(OR)

[P.T.O]

5. 	Write short notes on the following:											(5+5+5+5)

a.	Boundary layer thickness				b. 	Displacement thickness

c. 	Momentum thickness					d. 	Energy thickness

6.	a.	Using dimensional analysis derive an expression for forced convection heat transfer under

turbulent flow conditions. Also give the physical significance of the dimensionless

groups.							             										(16)

b. 	Write short notes on “Heat transfer in fluidized beds”		   						(4)

(OR)

7.  	a.	Derive an expression for Log Mean Temperature Difference for a parallel flow double pipe

heat exchanger. 							 									(10)

b.	Water at the rate of 3.783 kg/s is heated from 37.78 to 54.44oC in a shell and tube heat exchanger. On the shell side one pass is used with water as the heating fluid, 1.892 kg/s, entering the exchanger at 93.33oC. The overall heat transfer coefficient is 1419 W/m2oC, and the average water velocity in the 1.905 cm diameter tubes is 0.366 m/s. Because of the space limitations the tube length must not be longer than 2.438 m. LMTD correction factor F=0.88. Calculate (i) the number of tube passes (ii) the number of tubes per pass and (iii) the length of the tubes, consistent with this restriction. 								(10)

8. 	a. 	Briefly explain the flow patterns in agitated vessels.		         	   				(5)

b. 	How would you prevent swirling in agitated vessel 	        		   				(5)

c. 	Explain the various types of impellers used for Agitation		  					(10)

(OR)

9.	a.	Name any two mixing equipments used for mixing solid particles or Powders. 		(2)

b. 	Write short notes on “Geometric and Dynamic Similarities”.						(3)

c. 	Describe the various types of Separation processes with examples.	     				(15)