Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	DATA AND COMPUTER COMMUNICATION		              Time : 3 hours

Subject Code: 	CS248			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	List out the categories of Noise.

2.	------------------------------- provides a remote logon capability, which enables a user at a terminal or personal computer to logon to a remote computer and function as if directly connected to that computer.

3.	--------------------------- is the process of transforming analog data into digital signals.

4.	Give the HDLC frame format.

5.	------------------------- allows several transmission sources to share a larger transmission capacity.

6.	What are the key requirements for a routing function for a packet-switching network?

7.	---------------------------- is defined as the ratio of throughput to delay.

8.	In the ------------------ LAN Topology, each station is directly connected to a central node.

9.	------------------------------------- Flow control is a function performed by a receiving entity to limit the amount (or) rate of data that is sent by a transmitting entity.

10.	------------------------ is basically an unreliable and connectionless service; delivery and duplicate protection are not guaranteed.

PART – B  (5 x 3 = 15 MARKS)

11.	What is signal-to-noise ratio?

12.	Give the IEEE Definition of Error burst.

13.	State the advantages of using three-stage space division switch over single-stage cross bar matrix.

14.	List out the features of Layer2 switches.

15.	Justify the reasons for fragmentation.

PART – C  (5 x 15 = 75 MARKS)

16.	Analyze TCP / IP architecture in detail.

(OR)

17.	Explain Channel capacity in detail.

18.	Explain the three basic modulation techniques for transforming digital data into analog signal.

(OR)

19.	Explain different Automatic Repeat Request error control mechanism in detail.

20.	Explain Frame Relay protocol Architecture.

(OR)

21	Explain different routing strategies in packet switched network.

22.	Explain different Frame Relay congestion control techniques.

(OR)

23. 	Explain Bridges in detail.

24.	Explain the operation of connectionless Internetworking in detail.

(OR)

25.	Explain in detail how does TCP congestion control carry out.