Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	LINEAR INTEGRATED CIRCUITS AND APPLICATIONS    Time : 3 hours

Subject Code: 	EC211			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What are the different IC packages?

2.	Integrated resistors can be fabricated by __________, _________, _________or ________method.

3.	Give the voltage gain for a non inverting amplifier.

4.	Define slew rate.

5.	What is the frequency of oscillation for a phase shift oscillator?

6.	What are the limitations of the three terminal regulators?

7.	The two types of band pass filters are __________and _________.

8.	State the applications of monostable multi vibrator.

9.	Define capture range of a PLL.

10.	Which one of the D/A converter consumes the least power?

PART – B  (5 x 3 = 15 MARKS)

11.	What are the two techniques to provide electrical isolation between the different components and its interconnections?

12.	What are the characteristics of an ideal OP AMP?

13.	Give any two applications of comparators.

14.	Give the frequency response for different values of damping coefficient in a low pass filter.

15.	Give any two types of

a.	Direct type A/D converter		b.	Integrating type A/D convertors

PART – C  (5 x 15 = 75 MARKS)

16.	What are the basic processes to fabricate IC’s using silicon planar technology. Discuss each in detail.

(OR)

17.	Explain in detail the steps involved in fabrication of polysilicon gate enhancement NMOS.

18.	What are the dc characteristics that add error to the output of an OP AMP. Explain any two of them in detail.

(OR)

19.	Draw the circuit of an OP AMP differentiator. Explain. How can its stability be improved. Obtain the frequency response.

20.	Draw the circuit of an inverting Schmitt trigger. Also draw its transfer characteristics.

(OR)

21.	Explain in detail about the series OP AMP regulator.

22.	a.	Derive an expression for the transfer function of a first order low pass filter. Also derive the 	expression for frequency response.											(10)

b.	Determine the order of a low pass butterworth filter that is to provide 40 dB attenuation at

w/wh = 2.																	(5)

(OR)

23.	Give the functional description of a 555 timer and also explain how it works as an astable multivibrator.

24.	Explain in detail the working of an R – 2 R ladder type of D/A convertor.

(OR)

25.	Explain in detail the working of the successive approximation type of A/D convertor.