Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	SIGNALS AND SYSTEMS							              Time : 3 hours

Subject Code: 	EC227			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Define periodic discrete time signals?

2. 	Whether the system represented by y(n)=nx2(n) is linear?

3. 	Define a system

4. 	Write the difference equation governing the Nth order LTI system

5. 	What are the different types of signal representations?

6. 	State periodicity property with respect to DFT.

7. 	Assume two finite duration sequences x1(n) and x2(n) are linearly combined such that

x3(n)   = a x1(n) + b x2(n). What is the DFT of x3(n)?

8. 	The Fourier transform of a signal x(n) is _____________

9. 	What is meant by ROC?

10. 	ROC of a causal signal is the __________ of a circle of some radius r.

PART – B  (5 x 3 = 15 MARKS)

11. 	Explain the classification of signals?

12. 	What is Nyquist rate? Find the Nyquist rate of the following signal x(t)=2cos(1000t+π/4)+sin(2000t+ π/4)?

13. 	Explain the relationship between z-transform and DTFT?

14. 	Calculate DFT of the sequence x(n)={1,1,2,2}

15. 	Find the z transform and roc of -anu(-n-1)

PART – C  (5 x 15 = 75 MARKS)

16	a.	Test whether the following signals are periodic or nor and if the signal is periodic calculate

the fundamental time period.	                          								(5+5)

x[n]=2cosπ4n+sinπ8n

i)

x(t)=3cos5t+π6

ii)

b.	Comment on the following statement: “The sum or Product of two symmetric signals have any symmetry”	       														(5)

(OR)

17.	Prove the following.

- δ(n) = u(n) – u (n-1)

- u(n) = ∑ δ(k) ,     −∞≤K≤+∞

- u(n) = ∑ δ(n-k),   −∞≤K≤+∞

[P.T.O]

18.	a.	Find the Fourier series representation of the signal shown in fig-a and plot amplitude and      phase

spectrum for different values of T.

(OR)

19.	a. 	State and prove the properties of Fourier series										(9)

b.	Determine the Fourier series coefficients for. (i) f(t)=x(t)-y(t)							(3)

(ii)g(t)=x(t)+y(t)							(3)

20.	Explain the reconstruction of CT signals form its samples. Also comment on the effects of under sampling with necessary figures.

(OR)

21.	a.	Determine the response of the relaxed system characterized by the impulse response,

h(n)=u(n)to the input signal x(n)= 2nu(n)	    										(7)

b.	State and prove the properties of convolution		    								(8)

22.	a.	Derive the relationship between DFT and Z-Transform.								(5)

b.	Determine the sequence x(n) from X(k) = {4, 1-j, -2, 1+j}							     (10)

(OR)

23.	a.	Write notes on energy and power signal and calculate the energy of the sequence

x(n) = (0.5)n , n 0. 			         		     								      (4+4+4)

b.	Determine the fourier transform of unit step sequence x(n)=u(n).						(3)

24.	a.	Determine the Z-tranform for x(n)=n an u(n). Also determine its ROC.					(8)

b.	Find the z- Transform of x(n) = cosωn.  					       							(7)

(OR)

25.	a.	Determine the Z-transform and ROC of the signal,

x(n) = [ 3. (4n ) – 4. (2n)] u(n)														(8)

b.	State and prove time shifting property of Z transform.									(7)