Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	DIGITAL SIGNAL PROCESSING					              Time : 3 hours

Subject Code: 	EC228			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Give the graphical representation of unit impulse sequence.

2.	Define Discrete time signal.

3.	What is meant by frequency response?

4.	Determine the Z-Transform of unit step sequence.

5.	Draw a 2 – point radix – 2 DIT FFT butterfly structure.

6.	What is the advantage of FFT.

7.	What is butter worth approximation?

8.	Write the magnitude function of low pass butter worth filter.

9.	List the different types of window function used in FIR filter.

10.	Mention two transformation to digitize an analog filter.

PART – B  (5 x 3 = 15 MARKS)

11.	Determine the response of the following systems to the input signal

x (n) = { …..0, 3, 2, 1, 0, 1, 2, 3, 0 ……..}

a.	y (n) = x (n + 1)

b.	y (n) = 1/3 [x (n+1) , x(n) , x (n-1)]

12.	State and explain the time shifting (delay) property of Z-Transform.

13.	Define DFT and IDFT.

14.	How the order of the filter affects the frequency response of chebyshev filter.

15.	Define Bilinear Transformation.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain briefly about the classification of discrete time signals.

(OR)

17.	Explain the FM stereo system with block diagram.

18.	a.	Test if the following systems are causal or non causal

i.	y (n)	=	x (n) – x (n – 1)

ii.	y (n)	=	a x (n)

iii.	y (n)	=	x (-n)

b.	Test if the following systems are linear or non linear.

i.	y (n)	=	n x (n)

ii.	y (n) 	=	x (n2)

(OR)

19.	a.	Explain the convolution property of Z – transform.

b.	Compute the convolution of the signals

x1 (n)	=	1, -2, 1

x2 (n)	=	1 	0  n  5

=	0 	otherwise

[P.T.O]

20.	Find the DFT of a sequence x (n) = {1, 2, 3, 4, 4, 3, 2, 1} using DIF – FFT algorithm.

(OR)

21.	a.	Explain the circular convolution.

b.	Find the circular convolution of the two sequencer.

x1 (n)	=	{1, 2, 2, 1}

x2 (n)	=	{1, 2, 3, 1}

22.	Explain the chebyshev filter approximation in detail.

(OR)

23.	Write short notes on:

a.	Sample and Hold circuit		b.	Successive approximation A/D converter.

24.	Explain the various window technique used in design of FIR filter.

(OR)

25.	Find the DIRECT FORM I and DIRECT FORM – II realization of discrete time system described by Transfer function.

H(z)=8z3−4z2+11z−2(z−14)(z2−z+12)