Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	ELECTROMAGNETIC FIELDS						              Time : 3 hours

Subject Code: 	EC233			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What are the base vectors for Cartesian coordinate system?

2.	Define Gauss Law

3.	What is the relationship between Magnetic field intensity and Magnetic flux density?

4.	The unit of moment of force is _________

5.	What is the relationship between Current I and Current density J .

6.	Find the capacitance of a parallel plate capacitor with dielectric of εr =3 and area 0.92m2 with a separation of 4.5mm.

7.	Define Magnetization.

8.	Give the boundary condition for tangential component

9.	Write the Maxwell’s equation in point form or differential form derived from Faraday’s law.

10.	Compare Field theory with Circuit theory.

PART – B  (5 x 3 = 15 MARKS)

11.	Use spherical coordinates and integrate to find the area of the region 0≤φ≤ α on the spherical shell of radius a. What is the area if α=2π?

12.	A 200 turn rectangular coil 30cm by 15cm with a current of 5A is in uniform field B=0.2T. Find the magnetic moment m.

13.	Mention three properties of dielectrics.

14.	On the basis of magnetic behavior classify magnetic materials.

15.	State Faraday’s law of Electromagnetic induction.

PART – C  (5 x 15 = 75 MARKS)

16.	Calculate field intensity at a point on a sphere of radius 3m if a positive charge of 2 μC is placed at the origin of the sphere. Use spherical coordinate system.

(OR)

17.	Derive the expression for D due to a point charge and hence deduce the relationship between D and E.

18.	Using Ampere’s force law find H due to infinitely long straight conductor.

(OR)

19.	List the applications of Biot –Savart Law.

20.	State and explain continuity equation of current in integral form.

(OR)

21.	Determine whether or not the following potentials fields satisfy the Laplace’s equation a)V=x2-y2+z2     b)V= rcosφ +z     c)V= rcosθ +φ.

22.	Derive different expressions for energy density in Magneto static fields.

(OR)

23.	Calculate the inductance of a solenoid 8cm in length 2cm in radius having μr =100 and carrying 900 turns of wire.

24.	If the magnetic fieldH  =[3x cosβ +6 y sinα ] az. Find current density J if fields are invariant with time.

(OR)

25.	Define Poynting vector and derive the expression of Poynting theorem in integral form.