Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	MICROPROCESSORS AND APPLICATIONS		              Time : 3 hours

Subject Code: 	EC238			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The 8085 has __________ flags.

2.	Give two examples for register indirect addressing.

3.	IC ___________ is used to generate clock for 8086.

4.	The interrupts of 8086 are ___________ and _________.

5.	The direct data transfer between I/O device and memory is called__________.

6.	In 8255, the handshake signals used for input operation are ______and _____.

7.	The 8279 require an internal clock frequency of _________.

8.	Define Baud rate.

9.	Mention two applications of microprocessor.

10.	List two volatile semiconductor memories.

PART – B  (5 x 3 = 15 MARKS)

11.	Explain the difference between the conditional and unconditional return instructions. Give examples.

12.	Mention the special functions carried by the general purpose registers of 8086.

13.	Explain the different types of DMA.

14.	What are the features of 8259 that are programmed using ICW’s?

15.	What is resolution and conversion time in ADC?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Explain the different types of instruction sets of 8085 with an example. 	          		(12)

b.	Give the difference between wait state and bus idle condition. 						(3)

(OR)

17.	Draw the architecture of Intel 8085 microprocessor and explain.

18.	Explain minimum mode, maximum mode and common signals of 8086.

(OR)

19.	a.	Explain the different types of segment registers of 8086.		                     			(10)

b.	Write the flags of 8086.							          						(5)

20.	Draw the block diagram of programmable peripheral interface IC 8255 and explain the different

modes of operation with necessary diagram. Also draw the control word format of 8255.

(OR)

21.	a.	Explain synchronous and asynchronous data transfer scheme.                           		(10)

b.	Describe the important features of DMA controller.                                           		(5)

[P.T.O]

22.	With neat block diagram, explain the function of programmable interval timer. Also explain the

different modes of operation of 8253.

(OR)

23.	Explain the function of 8279 programmable keyboard/ display interface with its block diagram. Also explain the interfacing of 8279 with 8085.

24.	a.	Explain the memory organisation in 8085 based system.			          			(8)

b.	Describe the procedure of interfacing I/O and peripheral devices with

the microprocessor.	          													(7)

(OR)

25.	With neat sketch, explain the working of traffic light controller using microprocessor.