Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	MICROCONTROLLERS AND APPLICATIONS		              Time : 3 hours

Subject Code: 	EC239			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	How many register banks in 8051?

2.	Which registers are allowed to be used for register indirect addressing mode, if the data is in on chip RAM?

3.	The TCON register is not a bit addressable register. True or False

4.	Mention the ways to increase the band rate of data transfer in 8051.

5.	What is meant by wake up feature of 68HC11?

6.	What are the differences between a Harvard architecture and a Princeton architecture?

7.	Differentiate band rate and bit rate.

8.	What is the use of FOC bits in 68HC11?

9.	What are the two parameters to characterize each interrupt source?

10.	Give the format of ADC0N0 register used for channel selection.

PART – B  (5 x 3 = 15 MARKS)

11.	Write a program to find the sum of all 5 BCD numbers which are stored in RAM locations starting at 40H.

12.	Give the format of TMOD register and explain the operation of each bit in it.

13.	Discuss the memory space of Motorola 68HC11.

14.	Explain the working of free running counter as a watch log timer.

15.	Give and explain the register file structure in PIC microcontroller.

PART – C  (5 x 15 = 75 MARKS)

16.	Draw and explain the internal architecture of 8051.

(OR)

17.	Explain the group of instruction set in 8051 with examples.

18.	Describe the various operating modes of UART and associated control registers.

(OR)

19.	Explain the different I/O port structure hardware configuration with associated register functions.

20.	Explain the various port functions in Motorola 68HC11 at single chip mode operation.

(OR)

21.	Explain briefly about the following features of MC68HC11.

a) Real time clock timer			b) Pulse accumulator feature

22.	a.	Explain UART based asynchronous communication.

b.	Explain a synchronous serial communication with a separate clock signal.

(OR)

23.	Explain in detail about the feature SCI in 68HC11.

24.	Explain about stepper motor control using 8051 in detail with necessary diagrams.

(OR)

25.	Explain the Harvard architecture and pipelining process and memory allocation in PIC microcontroller.