Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	MICROWAVE AND OPTICAL COMMUNICATION ENGINEERING													               					Time : 3 hours

Subject Code: 	EC244			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the use of waveguide components?

2.	What is known as directional coupler?

3.	What is velocity modulation?

4.	The wave in the TWT is ------------------

5.	Read-type diodes are called as ----------

6.	Where the hybrid-pi equivalent model is commonly used?

7.	What is known as dominant mode?

8.	The refractive index of cladding is---------than refractive index of the core

9.	The radiance or ------of LED is a measure of optical power

10.	ThePIN diode is an------------

PART – B  (5 x 3 = 15 MARKS)

11.	What is known as a Magic tee?

12.	What are the two principles used in two cavity klystron?

13.	How the differential negative resistance is exhibited in IMPATT diode?

14.	What is a Snell’s law?

15.	What is known as Step index fiber?

PART – C  (5 x 15 = 75 MARKS)

16.	Explain Waveguide Tees in detail.

(OR)

17.	Derive S matrix of a Directional coupler device.

18.	Explain the principle of operation of two cavity klystron with its neat schematic diagram.

(OR)

19.	Explain the operation of Reflex klystron with neat sketch and applegate diagram.

20.	Explain the operation of BARITT diode in detail.

(OR)

21.	Explain the Bipolar microwave transistor with necessary diagrams.

22.	Explain wave propagation in Single mode fiber in detail.

(OR)

23.	Explain: 	a.	Modes               b.	Rays    		c.	Critical angle of incidence (5+5+5)

24.	Explain 	a.	Power Coupling  														  (7)

b.	Power launching versus wavelength,  in the  Source-to-fiber power launching (8)

(OR)

25.	Explain the PIN diode with a.	Schematic diagram 	b.	Energy-band diagram			(7+8)