Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	DIGITAL SIGNAL PROCESSING					              Time : 3 hours

Subject Code: 	EC245			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define a stable system.

2.	What are the applications of FFT?

3.	Write down the condition for anti symmetrical impulse response.

4.	Mention one property of FIR filter.

5.	Give the expression for the order of butterworth IIR filter.

6.	What are the procedures available to transfer analog filter into digital filter?

7.	What are the different types of quantization methods used in digital systems?

8.	What is the advantage of floating point arithmetic?

9.	Which architecture is used to access many memory cycles?

10.	What is the use of instruction RET in TMS processor?

PART – B  (5 x 3 = 15 MARKS)

11.	Obtain the frequency response of discrete time system with impulse function

h(n) = bnu(n) for |b|<1

12.	Draw direct form realization of FIR system

13.	What are the properties that are maintained same in the transfer of analog filter into a digital filter?

14.	Consider a second order IIR filter with H(z) = 1/(1-0.5z-1) (1-0.45z-1), find the effect quantization on pole locations of the given system function in direct form. Take  b=3 bits.

15.	Write about push and pop instructions used in TMS320 C5x processor.

PART – C  (5 x 15 = 75 MARKS)

16.	Find DFT of a sequence x(n) = {1,1,1,1,1,1,1,0} using DIF algorithm.

(OR)

17.	Find IDFT of a sequence x(n) = {4,1-j2.414,0,1-j0.414,0,1+j0.414,0,1+j2.414} using DIT algorithm.

18.	Using Frequency sampling method to design a FIR low pass filter with Wc= π/4 , for N=15.

(OR)

19.	Using a rectangular window, design a low pass filter with pass band gain of unity, cutoff frequency of 1000Hz and working at a sampling frequency of 5 KHz. The length of the impulse response should be 7.

[P.T.O]

20.	Design a digital Chebyshev filter that satisfies the following constraint using impulse invariance transformation. Assume T=1 s.

0.8≤H(ejw)≤1 ; 0≤w≤0.2π H(ejw)≤0.2 ; 0.6π≺w≤π

(OR)

21.	Obtain H(z) using bilinear transformation witnT-1 sec, for H(s) = s/ s2+2s+1

22.	Given H(z)= 0.5+0.4z-1 / 1-0.312z-1  is the transfer function of a digital filter  . Find the scaling factor S0 to avoid overflow in adder 1 of the digital filter. Draw its signal flow graph.

(OR)

23.	Write notes on Input quantization and Product quantization.

24.	Explain briefly the various addressing modes used in TMS320 C5X processor.

(OR)

25.	Explain about adaptive filtering.