Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	DIGITAL COMMUNICATION						              Time : 3 hours

Subject Code: 	EC249			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Write down the two types of uniform Quantizer.

2.	The frequency domain Nyquist criterion for distortion less base band transmission in the absence of noise is _____________.

3.	Quadrature phase shift keying used Coherent or Non-coherent modulation techniques.

4.	How many number of basis functions are used in M-ary PSK?

5.	Write down the impulse response of the matched filter in terms of Basis function.

6.	Give two examples for Non coherent modulation techniques.

7.	Define Information rate.

8.	Reed-Solomon codes are under cyclic codes. Say True or False.

9.	State Balance property.

10.	Mention the two types of spread spectrum Technique.

PART – B  (5 x 3 = 15 MARKS)

11.	State sampling theorem.

12.	Write down any three similarities of BPSK and BFSK.

13.	Draw the block diagram of non-coherent receiver for detection of binary FSK signals and explain.

14.	Calculate the Shannon channel capacity for a signal to noise ratio of 7 and bandwidth of 4KHz.

15.	Generate the PN sequence using the maximum length sequence generator as given below

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Draw the block diagram of differential PCM and explain the function performed by each

block.																		(10)

b.	Explain the transfer characteristic of a quantizer of mid tread type.					(5)

(OR)

17.	Discuss the demerits of duo binary signaling scheme. Explain in detail how the pre-coded duo binary signaling scheme is used to reduce the demerits.

[P.T.O]

18.	With necessary equation and signal space diagram, discuss the principle and operation of coherent binary FSK.

(OR)

19.	Draw the block diagram of QPSK system and explain it’s operation with signal constellation diagram.

20.	With a circuit diagram explain the operation of base band signal (binary) receiver. Also derive the peak signal to RMS noise output voltage ratio.

(OR)

21.	Discuss the operation of coherent BPSK receiver. Also derive its average probability of error.

22.	a.	State and explain Shannon’s theorem.											(8)

b.	Explain the entropy information rate.											(7)

(OR)

23.	Briefly explain about the generator matrix and parity matrix used in linear block codes with suitable example.

24.	Derive the signal space dimensionality and processing gain for a direct sequence spread spectrum.

(OR)

25.	What is frequency hopping? Explain with suitable diagrams, how the PN sequence is used in slow frequency hopping spread spectrum.