Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	VLSI DESIGN										              Time : 3 hours

Subject Code: 	EC255			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define Threshold Voltage.

2.	What are the basic CMOS technologies?

3.	What are the two common approaches used for Oxidation Process?

4.	Define Pass Transistor.

5.	What is the need for design rules?

6.	Define Sheet Resistance.

7.	Draw a CMOS two input NAND gate circuit?

8.	What is two phase clocking?

9.	Define PLA, CLA.

10.	Define the term Regularity.

PART – B  (5 x 3 = 15 MARKS)

11.	Write short notes on: 	(a)  Architectural Design     (b)  Logical Design	     (c)  Physical Design

12. 	Describe about delay unit and NMOS inverter pair delay?

13. 	Discuss about:       	(a) 	Area capacitance 			      (b) Wiring capacitance

14. 	Mention the difference between switch logic and gate logic?

15.  	Explain Pipelined multiplier array?

PART – C  (5 x 15 = 75 MARKS)

16.	Discuss N-well CMOS Technology and compare with twin tub process with neat diagrams.

(OR)

17. 	Discuss P-well CMOS Technology and compare with Silicon on insulator process with neat diagrams.

18. 	Explain the various Electrical properties of  MOS  Transister Circuit.

(OR)

19.	a.	Discuss the operation and characteristics of CMOS Inverter with neat diagram.

b.	Write a note on CMOS Fabrication.

20. 	Explain it briefly about Design rules for Silicon gate N-MOS  Process

(OR)

21. 	Explain briefly about Design rules for CMOS N-Well Process.

22.	Explain in detail

a.	Dynamic CMOS Logic	b. Clocked CMOS logic	c. CMOS dynamic shift register

(OR)

23.	Explain in detail

a.	Structural Design of Parity generator		b. Charge storage

c.	Precharged Logic.

24.	Explain the design of ALU Subsystem with an example.

(OR)

25. 	Explain how the ALU function can be implemented with an adder.